DATA PROCESSING IN MARKER-BASED MOTION ANALYSIS†

THOMAS, Júlia, JOBBÁGY, Ákos, FURNÉE, Hans, FOGARASI, Fábián

Dept. Measurement and Information Systems, Technical University of Budapest

1111 Budapest, Műegyetem rkp. 9. Phone: (+36 1) 463 2057, fax: (+36 1) 463 4112

web: http://www.mit.bme.hu,  e-mail: tjulia@mit.bme.hu

ABSTRACT

The important points of a moving object can be tracked by sticking markers to them. These markers are carriers; after acquiring a picture the excessively bright marker images should be extracted and processed. Different image processing algorithms are used to determine the position of a marker image in 2D. Based on the 2D images of at least two cameras the 3D positions can be calculated. The paper describes the rules we suggest for automatic tracking of markers and the possibilities to estimate the missing parts of trajectories. Early detection of Parkinson’s disease based on movement analysis is given as an example for the application of 2D analysis.

Keywords: motion analysis, passive markers, trajectory estimation.

1. LEVELS OF DATA PROCESSING

In marker based motion analysis, markers are used as “carriers”. It follows that we can divide the data processing into two parts: low-level data processing, what is required to get the co-ordinate values of the points where markers are attached to. The second part is the further processing of marker midpoint co-ordinates:

- lens distortion compensation,

- marker identification and tracking,

- user specific algorithms.

The co-ordinate values of a marker are determined on the basis of estimating the centre of its image. The most frequently used method is geometric centroid calculation [1], [2], [3]. There are methods that provide more accurate results even if the marker images are distorted [4], [5]. At least two cameras are needed to determine the position of a marker in 3D. The actual camera set-up must be calibrated using a marker pattern, where the distances between markers are known with high accuracy. The parameters for lens distortion compensation can also be computed during the calibration process.

Identification and tracking of markers is a problem during passive marker-based analysis. When there are several cameras in the measurement set-up ambiguous situations can be resolved. One example is given in Figure 1. In applications which require and/or can afford only a single camera, identification and tracking can be aided by making use of the a priori knowledge concerning the movement to be analysed. This method does not provide a general solution but is very effective if the same movement pattern is to be recorded and evaluated many times.

Figure 1. Marker M2 is hidden by M1 on the image of camera1. The situation is resolved by the image of

camera2

.

Figure 2 demonstrates that marker overlapping and occlusion can be resolved based on the images of a single camera if preceding and succeeding positions are also considered.

Size information of marker images can also help in sorting and tracking. Depending on the velocity of markers and the sampling rate of the camera the overlapping marker image can appear on several frames. Some examples are given in Figure 3.

Sorting and tracking in the single camera case is detailed in the next section. When a marker is occluded by an object a part of the trajectory is missing. Making use of a priori knowledge and of the known positions different estimates can be given for the missing part. This is also detailed in section 2. A neurological application is described in section 3, which is a good example for the application of a single camera motion analyser.

Figure 2. Marker overlapping can be resolved using a single camera if preceding and succeeding positions are also considered.

Figure 3. Overlapping marker images.

- SORTING AND TRACKING

Sorting of markers in image-based motion analysis means the identification of the bright patches found in single frames. Marker trajectories are assembled on the basis of the sorted frames. The corresponding size information of the bright patches is important in the identification process of the markers.

The position of an occluded marker cannot be determined directly. A marker can be occluded by another marker. In this case the images of the two markers are identified as only one bright area with only one centre point. In the case of complete occlusion a single bright area appears and the same centre point is related to both markers in that frame. If markers overlap partly, the calculated centre point of the resulting bright area is different from the centre points of the real marker images. The resulting whole bright area is usually assigned to the marker that is closer. Thus results can be on the one hand incorrect, on the other hand incomplete, i.e. wrong or missing parts of trajectories can occur.

Bright or reflecting objects appearing in the background, e.g. wrist-watches or decoration, can be falsely identified as markers (ghost markers). Often this can be excluded by setting an appropriate threshold level during digitisation. If ghost markers still appear they are usually present over a period of a few frames only. This results in short trajectories, which do not belong to the analysed movement and have to be excluded from the analysis. Here, the size of detected bright areas plays an important role, because bright areas of such objects are usually significantly smaller than bright areas of markers.

When several markers are used, a movement can be recorded correctly only if the markers are identified and sorted in each frame. When passive markers are used, sorting cannot always be done unambiguously. Tracking means that for each bright patch of a marker we find the appropriate bright patch in the next frame.

Generally, for precise motion analysis 3D-recording of the movement is necessary. When using a single camera only 2D co-ordinates can be extracted, depth information (z co-ordinate) is missing. At least 2 cameras are needed to determine 3D positions of markers. In case of crossing trajectories or when a marker disappears in a frame, a set of cameras can offer a possibility for identification.

For building marker trajectories different algorithms are available. The simplest method is the nearest neighbour association. This procedure uses the fact that in consecutive frames the position of a marker image changes only slightly. Therefore, on the basis of the position of a marker image, in the following frame the closest bright patch can be assumed as the image of the same marker. [6] gives a good summary of tracking algorithms.

The method of maximal speed can be applied if a priori information regarding the speed of the movement is available. From the speed vmax and the sampling rate fm the maximal distance rmax.speed. can be calculated, which is the maximal distance to be covered by a marker in 3D, until the next frame. The algorithm calculates a circular area with the radius rmax.speed. around the position of each identified marker. The position in the next frame for a marker is searched within this area.

The extrapolation method uses the a priori knowledge of the kinetics of the regarded movement. The maximal acceleration of the markers is usually known. Based on the previous positions of a marker the estimation of its position in the following frame can be calculated by means of extrapolation. Around the estimated position a circular area is determined with radius rextrapol., and the bright patch found in this area is assumed to be the next marker position. Figure 4 shows the difference between the circular areas set by the method of the maximal speed and extrapolation method.

Figure 4. Circular areas for the method of max. speed and for the extrapolation method.

At the beginning of a trajectory, when no information for extrapolation is available, the identification must be executed by other methods, e.g. with the procedure of max. speed. The missing positions can be estimated by means of extrapolation. On the basis of the movement the maximal length of the missing trajectory should be specified. When having a long gap in the trajectory, the estimation of the real marker movement can be unacceptable. When sequential positions are estimated by means of extrapolation, the search area should be increased after each step due to increasing uncertainty in position estimation. As an advantage, a bright patch, which is outside of these search areas, cannot be the image of a reappearing marker.

Table 1 shows a comparison of the algorithms for building marker trajectories under non-ideal conditions.

| Method | occluded marker | ghost-marker | occluded marker + ghost-marker |
| --- | --- | --- | --- |
| Nearest neighbour | No opportunity to estimate the actual marker position. | Relatively bad recognition of ghost-markers | high probability of false identification |
| Max. speed | No opportunity to estimate the actual marker position. | Recognition of ghost-markers with high probability | small probability of false identification |
| Extrapolation | Good opportunity to estimate the position of an occluded marker. | very good recognition of ghost-markers | very small probability of false identification |

Table 1. Comparison of the methods for building of trajectories

Frequently, moving segments and therefore attached markers can move only within certain ranges. This leads to a limitation of the area in which a marker can appear. During gait analysis the head or hip can move only within a certain range of height, cf. Figure 5.  The Dyhead and Dyhip heights have to be determined for each person individually. In case of a reappearing marker, which has been covered in several frames, the identification is greatly helped by this a priori knowledge.

Figure 5. Markers can appear only within specified ranges during gait analysis.

Another help for marker identification results from the fact, that the distance between certain markers cannot change. This is applicable, for example, during the analysis of human movements to pairs of markers such as wrist-elbow, elbow-shoulder, foot joint-knee pairs etc. This method is particularly applicable during 3D-tracking. In case of 2D-analysis it has to be noted that the distance between the projected images of a pair of markers varies as a function of their distance from the camera and the projection angle. With such methods many false identifications can be excluded during marker tracking. Estimation of missing trajectory parts is detailed in [7].

If markers overlap or if they are occluded, then there will be frames, in which no proper marker identification can be done. Based on the known positions the missing trajectory parts can be estimated with different methods, e.g. with the Bernstein-Bézier or the B-Spline-method. In case of overlapping markers the resulting whole bright area will be assigned to the closer marker if sorting is based only on a single frame. Consequently, there will be a step in the calculated trajectory. The steps have to be smoothed, in order to get appropriate data for motion analysis. For this task there are different algorithms, e.g. the Smoothing Splines algorithm or the Interpolative refinement.

While comparing the algorithms gaps of different lengths were inserted into trajectories. For small gaps (length up to 10 frames) the positions of the marker images can be interpolated with small deviation, while larger gaps (over 30 frames) can be estimated only with significant errors. The B-Spline method has a smaller interpolation error than the Bernstein-Bézier method, but in case of a small gap this does not significantly influence the trajectory. In case of a large gap, the Bernstein-Bézier method estimates the missing part of the trajectory only roughly, while the B-Spline method gives a good estimation of the missing part.

If there is an abrupt change in the trajectory during the missing part, only a rough estimation of the movement can be given based on square B-Splines. As a consequence, the actual movement cannot be reproduced correctly. The resulting trajectory estimation is a straight line between the edge points. In general the Bernstein-Bézier algorithm fills a larger gap than the B-Spline-method with the same error. Further evaluations are given in [8].

- DATA PROCESSING IN A NEUROLOGICAL APPLICATION

Movement patterns are characteristic for a person. Changes in the movement patterns indicate changes in the physical state of a person. Several diseases distort the movement patterns of patients. Usually the deviation starts with subtle changes that can hardly be detected by human observation. We have been using a precision passive marker based motion analyser, the PRIMAS [9] to record the movement patterns defined by neurologists. Our aim is to define a screening test for the Parkinson’s disease. We specified a set of movement patterns, that helps in establishing the diagnosis for patients, whose condition is not previously known.

Markers are attached to the fingers or to the arm of persons and the position - time functions of the markers are recorded. The following parameters have been found to be relevant in the case of finger, hand and arm movements: smoothness, symmetry, frequency, amplitude, and the effect of the movement of the other hand. Feature extraction is especially difficult when the two hands perform different movements.

The test we suggest can be completed in 15 minutes. Detailed description of the movement patterns and the evaluation are given in [10] and in [11]. We recorded the movement patterns of 12 young (20...25 years old), 10 senior (50...65 years old) healthy subjects and 9 Parkinsonian patients (50...65 years old). Based on the evaluation of all movement patterns a global parameter was calculated for each person. The results are given in Figure 6. Though the global parameter for a group shows substantial variations the maximum for the healthy group is less than half of the minimum for the Parkinsonian group. The tested Parkinsonians all belonged to stage I or II of the possible I...V, according to the Hoehn-Yahr scale.

For the evaluation the records were segmented and average cycles were processed. The aim was to make a binary classification, whether or not the tested person was effected by the disease. Currently, there are new algorithms under development. We plan to characterise the actual state of patients on a continuous scale. This can give a feedback to the neurologist regarding the effectiveness of treatment and medication. The question is, whether continuous scaling is available, when movement cycles are evaluated independently. We suspect, that there was a considerable amount of information loss by the averaging of the data and by using frequency domain analysis. Whenever there was an averaging, it acted like a low pass filter and valuable details were lost. Fourier transforms also generated results describing the whole record, not the separate cycles. If these methods could be avoided, and consecutive cycles were to be treated independently, more details would be seen as a result of the evaluation.

Figure 6. Global parameters of tested persons.

For the rest of the paper one of the predefined movements, namely tapping, will be used as a demonstrative example. In this exercise, the hands of the subject rest on the table according to Figure 7. The task is to imitate tapping by first striking the table by the little finger, next by the ring finger and so on, while leaving the palms and the thumbs still. The movement should be carried out as fast as possible.

Figure 7 Tapping test.

Up to now, the evaluation was based on finding a dominant frequency for each finger and these values were compared to one another. Their similarity determined the action to be taken. This method was sufficient for differentiating healthy subjects from Parkinsonians. However, the progress of the disease could not be classified with high resolution.

In order to increase the resolution, the autocorrelation of the movement of the fingers was considered instead of the frequency spectrum. The theory behind it is that tapping is a quasiperiodical movement, when carried out by a healthy subject. On the contrary, it becomes much more disorganised, when a patient with the Parkinson’s disease performs it. It is of neurological reasons, that  quasiperiodicity is lost at one or both hands at these patients. Thus autocorrelation is not only able to detect the difference in the movement patterns of these two groups, but also provides a parameter with continuous range of values. Figure 8 shows a fraction of the recorded movement and its normalised correlation function for the index fingers of the right and the left hand of a patient. His disease is in between stage I and II, when it is typical to have unilateral symptoms. Figure 8 demonstrates that only one hand shows periodic movement with a period of about 200 ms.

Figure 8. The position-time function (left) of the index fingers of a Parkinsonian patient and the corresponding normalised correlation functions (right).

- REFERENCES

- Taylor, K.D., Mottier, F.M., Simmons, D.W., Cohen, W., Pavlak, R. jr., Cornell, D.P., Hankins, B. (1982): An Automated Motion Measurement System for Clinical Gait Analysis. J. Biomech., 15#7, 505-516.

- Winter, D. A., Quanbury, A.O., Hobson, D. A., Sidwall, H. G., Remer, G., Trenholm, B., Steinke, T., Schlosser, H. (1974): Kinematics of normal locomotion - a statistical study based on T.V. data. J. Biomech., 7, pp. 479-486.

- Dinn, D.F., Winter, D. A., Trenholm, B. G. (1970): CINTEL - Computer Interface for Television. IEEE Trans. on Computers, Vol. C-19, pp. 1091-1095.

- Jobbágy, Á., Gyöngy, L., Martin, F., Ábrahám, Gy. (1997) Processing of images in passive marker based motion analysis. Periodica Polytechnica, Ser. El. Eng. Vol. 41. No. 1. pp. 63-74.

- Ferrigno, G., Pedotti, A. (1985): ELITE - a digital dedicated hardware system for movement analysis via real-time TV signal processing. IEEE Trans. on BME, vol. 32., pp. 943-950.

- van der Schaaf, A. (1993): Stereometric data association in motion analysis. An implementation of multiple hypothesis data association for PRIMAS. MSc Thesis, Delft University of Technology, Fac. Applied Physics, Section Signal Processing.

- Shirai, Y. (1987): Three-Dimensional Computer Vision. Springer Verlag, Berlin - Heidelberg, 1987.

- Thomas, J. (1998): Identifikation und Verfolgung von Marken in Bildfolgen von bewegten Körpern. Diplomarbeit, Universität Karlsruhe, Fakultät für Informatik, Institut für Algorithmen und Kognitive Systeme.

- Furnée, E.H., Jobbágy, Á.(1993): Precision 3-D Motion Analysis System For Real-time Applications. Invited paper, Microprocessors and Microsystems, Vol.17., Nr.4. May 1993. pp. 223-231

- Jobbágy, Á., Furnée, E.H., Harcos, P., Tárczy, M.: Early Detection of Parkinson’s Disease through Automatic Movement Evaluation. IEEE Engineering in Medicine and Biology Magazine, Vol. 17. No. 2. March/April 1998. pp. 81-88.

- Jobbágy, Á., Furnée, E.H., Harcos, P., Tárczy, M., Krekule, I. and Komjáthi, L.: Analysis of Movement Patterns Aids the Early Detection of Parkinson’s Disease. 19th Annual International Conference of the IEEE Engineering in Medicine and Biology Society, 30 October - 2 November, 1997, Chicago, IL. USA, pp. 1760-1763. the proceedings on CD-ROM.

† The research work is supported by the Hungarian Scientific Research Fund (OTKA), T 23125 and the Netherlands Organisation for Scientific Research, 048.011.022.