Some issues on AI techniques in RT process control

# P. Albertos, J. Picó, J.L. Navarro and A. Crespo

Dept. of Systems Eng., Computers and Control

Universidad Politécnica de Valencia, Spain

e.mail: pedro@aii.upv.es

Abstract.    The use of AI techniques in control raises the problem of implementing operations with not well delimited computing time, in a real time framework. In this paper, the combined used of conventional and AI control strategies at the lower levels of process control is analized. The main time constraints as well as some structures, namely the integrated and the hierarchical control structures, are reviewed. Two applications in the control of industrial kilns are also discussed.

Keywords. Real time control, delays, Intelligent control, Artificial intelligence, kiln control.

INTRODUCTION

To deal with the integral control problem in the process industry, where a number of different tasks are involved, the use of a bunch of control techniques is required. Other than the well established solutions based on the classical control theory, artificial intelligence (AI) techniques are also becoming a common practice to develop control systems for local and global control, adaptation, learning, supervision and coordination, optimization, planning and even system management.

Classically, the integral control problem may be decomposed into different levels, as shown in fig. 1, each one dealing with different goals and kind of information. From the real time point of view, the time constraints are also quite different.

At the first level, the local control is implemented based on information directly got from the process. Typically, PID controllers or simple automata cope with most of the goals. The control viewpoint is very close to the process and there is almost one control loop per variable.

Optimal and interactive control have a multivariable structure but the approach to the control problem is quite similar: an algorithm provides the control action based on numeric data. Both, the control structure and the parameters are fixed.

At the second level, the control parameters are automatically tuned by means of adaptation laws, also operating in an algorithmic way. This strategy allows the control of time varying or non-linear processes.

The supervision, also involving local optimization algorithms, may change the controller structure, the set points or the global control strategy. At this level, some heuristic is required. It may be obtained either directly or by reasoning procedures. In the case of multimode operating processes, the supervision system allows the use of the most suitable controller among a set of predefined ones.

PROCESS

Local Control

Adaptation

Supervision/

Optimization

Coordination

PID

s.p.

Obj./Restric.

Operator/Other systems

Information

Qualitative

Numeric/

Logic

Numeric/

Statistic

Statistic/

Heuristic

Levels

parameters

Fig. 1. Integral Control Structure

At the top level, where decisions about the goals, the processes, sequences and so on are taken, a lot of qualitative information is involved.

For the last twenty years, several proposals have come out concerning new techniques trying to improve the performances provided by the controllers by endowing them with facilities so far considered as belonging to the human domain. Learning, symbolic reasoning and pattern recognition are the most representative. Most of these techniques come from the Artificial Intelligence field, adapted to the intrinsic characteristics of control systems. This has opened a new broad research line, the so-called Intelligent Control.

Typically AI techniques have been applied to activities at the top levels: expert systems, off-line optimization, diagnosis, decision-making, and so on. In fact, these techniques are suitable to deal with: 1) Fast full information from non-linear and complex models (Neural Nets), 2) Static and dynamic approximated knowledge (Fuzzy Systems), 3) Heuristics in both model and goals, involving reasoning (Expert Systems), 4) Optimization problems with multigoal complex specifications (Genetic Algorithms), as well as a mixture of all of them.

The use of these new control methodologies also reaches the lower levels due to the need of controlling systems with increasing complexity. Basically, in process control one can find three main causes for difficulties, [1]: inherent complexity of the system, presence of nonlinearities, and presence of uncertainties. Facing them requires to modify the traditional concept of control, so as to include features such as decision taking, plan generation, learning, etc., required to design systems with a high degree of autonomy [2].

In this context, control system reaction can be computed as result of either, evaluating rules (reasoning), or ``unconscious'' reflex reactions, based on the process behavior. Intelligence, thus, stands for the    capability to reason on facts and rules and/or the capability to learn about the system behavior [3]. There are a number of industrial applications already running satisfactorily, [4].

All these control activities should be implemented in a common framework, competing to get control of the system resources and, as previously mentioned, accomplishing some, more or less restrictive, time constraints, including: a) Delays in providing conclusions, b) Getting on time response, c) Performing refinements and learning, d) Dealing with distributed systems, e) Synchronism problems and f) Emergency and degraded functioning.

The structure of this paper is as follows. First RT control issues, in both algorithms and software implementation, are summarized. Then, algorithms and schemes involving AI control are reviewed, focusing the attention on the time requirement as well as their suitability for some basic steel industry processes. Different techniques to implement classical/AI cooperative control systems are discussed. Finally, two already developed applications in the cement and the ceramic industries are reported and some conclusions are drawn.

##### Computer Direct Digital Control

In a complex control scheme the existence of various processes (multiprocess), with many control variables and sensors (multivariable), involving many control loops (multiloop), trying to reach many objectives (multigoal), with different kind of data and decisions (multilevel) and operating at different sampling rates (multirate) must be considered. The general problem can be approached in different ways: 1) assume everything is independent, develop control algorithms and apply them at the scheduled time, trying to minimise delays and interactions, 2) assume the actual situation and develop control algorithms based on the full and detailed process model, 3) generate software mechanisms to make available complementary data, 4) provide different levels of service: the fastest for safety operation, the slowest for routine activities, and an optimized intermediate level for normal system operation, 5) combine all the available data and control strategies in the best possible way.

In these processes, the assumption of regular and uniform sampling as well as a simple algorithmic running of the control operations are not more possible. In a recent paper [5], a review of the non-conventional sampling schemes is presented. In particular, algorithms for specific operating conditions are discussed, including variable sampling period, non-synchronous sampling, multirate sampling, missing data, and event driven control.

In summary, the leading issues are: i) to compute the control action based on the latest information obtained from the process and updated to the moment of delivering this control action, by means of the process model, ii) to shorten the time interval or delay between control, and measurement samples (the control is blind) by re-structuring the computer implementation of the control algorithms, and iii) to reduce the control updating interval (the control system is open-loop) according to the process dynamics by also minimizing the global control period as a result of a reduction in mandatory tasks per period, as discussed in the next section.

To fulfill these constraints, modified classical control algorithms, like PID or state feedback combinedwith advanced observers, have been proposed [5].

In most of these cases, the control law is predetermined and the computation time is easy to compute. This is not the case for AI techniques, where optional activities with real time constraints should be considered.

## RT Constraints and operation

To develop an integral control application a system design able to guarantee the timing constraints of the different processes is needed. The AI based control techniques, such as NN, FLC, rule-based systems, etc., allow to better capture a system model and apply the appropriated techniques to each problem or subproblem but, in most cases, their execution time is unbounded or unpredictable.    So, it is required to modify the classical concepts of tasks or activities to work with more flexible systems.

There are control system activities with different kind of time constraints.

Hard time constraints refer to those for which a solution must be provided at or before a required time, otherwise the system integrity is lost. Soft time constraints refer to activities which are relevant but if they are not executed, an acceptable or degraded system behavior is obtained. Some other activities should be done within a longer period of time but their timing is not so critic.

In a classical real time application several parallel activities have to be defined. Periodic tasks model the activities that must be executed at periodic instants of time. Sporadic tasks model the activities that handle external significant events. All these tasks are concurrently executed under the supervision of a scheduler. In a basic control loop implementation, Audsley [6] proposed the control computation to be composed by three blocks: a mandatory, an optional and a final part. If AI techniques should be considered in the model, a complete control loop implementation would have the following scheme:

- Computation of the control action. It is mandatory and should be executed as soon as possible. At the end of this part, the basic control action is ready to be sent to the actuators.

- Solution improvement. During this time, an algorithm or method is able to improve the answer obtained in the first part. This part can be considered as optional.

- Output the control action. Must be done either as soon as possible or within a fixed time interval.

- Evaluation of the global state and preparation of the data for the next period. This part should be executed before the next period, but at any time within the period.

These activities can be represented in a graphical way, fig. 2, where   represents the data-acquisition system delay and  the control/measurement one.

Fig. 2. Actions delay

One of the important aspects in the above model is related to the computation characteristics of part 1, 2 and 3. The first, third and fourth parts can be considered as the classical control loop. The second part introduces the possibility to incorporate new computation depending the time availability to improve the system result.

These aspects are shown in fig. 3.

Fig 3. Task scheme

Activities 1 and 3 must be executed before the deadline of the task. Activity 2 is optional and can be executed as far as there is available time.

The main concerns being to fix and keep as short as possible the time used in activities 1 to 3, and the timing between tasks, if they must share data and/or results.

For a number of parallel tasks, competing to get CPU access, some issues should be considered. Each task will have a different priority but many of them will be at the same level. One of the software engineering practices is to determine if the application is schedulable or not based on a few parameters, that is, if the time requirements are guaranteed. However, even if the system is schedulable, some action delays can modify the system behavior. The interval of possible variation of the actuation, the Control Action Interval, (CAI) is defined as the time interval where the action is sent to actuators at different periods.

If the number of tasks increases, the CAI of the low level priority tasks is also increased. Although from an application viewpoint the priorities should be related to the relevance of the activity, from a software viewpoint the priority assignation is basically determined taking into account the period of the task. A higher priority is assigned to more frequent task than to less frequent ones. Because of the CAI depends on the interference level with other tasks, if the priority level of a task is increased or decreased, its CAI will be consequently reduced or increased.

However, from the scheduling theory point of view the priority of a task should not increased or decreased. In order to evaluate the schedulability, a priority is attached to each task depending on its rate or deadline. A first solution is to decrease the deadline of a task maintaining the period. When deadlines are shorter than periods, the deadline monotonic priority assignation can assign a higher priority to the modified task. Using this approach, the system is stressed and the modification can produce a non    schedulable system.

Another approach that does not stress the system consists in the period modification. So, the change of the task priority should be a consequence of the change of the period. From the control point of view, we can not change the period, but from the software design point of view, we can apply some techniques to change the period of a task. This change will produce a priority .

To increase the priority of a task, we could split a task into two parts, dividing by two the period and executing alternatively a part each period.

An alternative method consists on increasing the period of a less important task from the user point of view, but more frequent than other which are more important. In any case, the reduction of the CAI of a task produces an increment in the CAI of another one.

##### BASIC AI TECNIQUES IN CONTROL

Let us review the basic AI techniques as well as some applications in the steel industry.

- Neural Networks

A broad descriptive definition of Artificial Neural Network (NN), inspired in their biological counterparts, may simply formulate a NN as the structure outcoming from the interconnection of basic processing units, or neurons, each of them performing a nonlinear operation on its inputs.

From the neurocontrol point of view, a NN may be considered as a convenient parametrization of a family of nonlinear mappings, capable of representing general nonlinear systems [7] under certain assumptions. Therefore, they may be considered like another function approximation tool, but endowed with some interesting advantages [8].

Out of the many possible neural network topologies devised over the last thirty years, only three of them are widely used for neurocontrol purposes, for they represent suitable nonlinear function parametrizations:

•	Multilayer perceptron, or multilayer feedforward NN (MFNN).

•	Radial basis function NN (RBFNN).

•	Recurrent, or dynamic network (RNN).

From the point of view of the time requirements, two aspects can be considered regarding the implementation of NN:

- Parallel distributed processing. As most of the operations involved in the NN processing are repeated over many nodes, parallel (vectorial) processors or transputer arrays may lead to very efficient software implementations.

- Dedicated hardware implementations. The referred parallel processing of simple elements (neurons) can be implemented in specific VLSI hardware leading to dramatic speed improvements over its software    counterparts (moreover if compared with non-parallel CPU's) when applied to challenging real-time    problems. Once a problem is solved with a network, incrementing the number of neurons is the way to solve a similar one with more variables, so the processing time keeps constant even with increased dimensionality of the problem. Processing times in parallel hardware implementations depend on the number of layers and not on the number of nodes in each layer as these are processed all at a time.

Most of the times NNs are implemented as a piece of software code to be run on a sequential processor. In this case the processing time depends both on the number of layers and the number of neurons in each layer.

In any case the processing time is always bounded and can be known for all the implementation described. The only case in which the processing time may be variable is in the case of variable structure NNs implemented on sequential processors.

On the other hand, two different sequences of operations can be distinguished during run-time:

- Forward propagation of information. The NN processes its inputs, forwarding them through the layers, and outcoming its output. The time consumed depends on whether the NN is implemented in dedicated hardware, a parallel processor or a sequential processor. In all cases the processing time is fixed, and forward propagation of information is a compulsory task.

- Weights tuning. The learning of the correct weights requires backpropagating the information through the network, in order to obtain the required derivatives of the network error with respect to the network weights. Afterwards the weights are modified following some adaptation rule. On-line learning can be carried out following several time schedules:

- Learning at each sampling period. This will require that both forward and backward propagation, and the adaptation rule are executed at every sampling period. The last two are executed jointly, and their execution can be deferred to the final part of the sampling period.

- Periodic Learning, accumulated learning over a fixed number of periods. Forward and backward propagations are executed at each period, and the results are stored. At the end of the fixed interval, the stored results are used to update de weights. The backward propagation can also be deferred to the end of the interval. In this case a time-efficient vectorial implementation of the learning algorithm can be used.

- Sporadic Learning. Whenever a degraded performance of the control system requires re-tuning of the network weights, the backward propagation and adaptation rule are executed. Sporadic learning can also be carried out whenever time resources are available, even if not strictly required. This kind of learning should be carried out over the data corresponding to an interval spanning several sampling periods, to avoid problems caused by using a poor estimation of the gradient of the error with respect to the weights.

b) Fuzzy and neurofuzzy controllers

Fuzzy controllers have become one of the most successful applications of fuzzy logic for the last years, [9]. They can be easily applied to control processes of which we lack a useful mathematical model, or in which vagueness plays a fundamental role and we have qualitative models. The knowledge supplied by an expert human operator is coded in the form of IF-THEN production rules, where the premise and consequent variables are categorized in terms of fuzzy sets. At the controller output, the control action inferred by a set of fired rules is obtained by means of interpolative reasoning methods.

Fuzzy controllers with propositional rules, that is, those in which the consequent of a rule is never the antecedent of another rule are functionally equivalent, under the usual assumptions on the class of inputs and membership functions employed, to NNs with one layer [10,11]. Therefore, if the possibility of adapting the membership functions is assumed, all the considerations mentioned for NNs are valid.

Neurofuzzy controllers are but an implementation of a fuzzy system under the structure of a NN. Therefore, their time requirements are equivalent to those described for NNs.

The simplest control schemes are represented by single intelligent controllers or by estimators implementing a direct mathematical mapping. Fuzzy systems with simple propositional inferences, single NNs and neurofuzzy systems are typical representatives. To get most of the advantages of these new approaches, optionality about knowledge sources and procedures should be considered, introducing additional difficulties in RT control. Some of these new structures are discussed in the next sections.

The natural conception of expert systems as well as evolutionary programming techniques (like genetic algorithms) makes these approaches not suitable for RT applications, and will not further considered.

c) Steel Industry Processes

The steel industry involves a lot of different processes. Some of them are the basic steel production processes, such as the blast Furnace, the Soaking Pits, or the Rolling Mill, but there are also many other complementary processes, such as blending processes (in raw materials), recycling of waste, coating and treatment lines and so on.

Most of these processes fulfill the characteristics for AI techniques suitability: the state knowledge is approximate due to difficult measurement of relevant variables, the behavior is highly nonlinear, and there are joint goals to be reached. Some applications have been already developed. [12].

Blast Furnace

In steel making processes, high temperature and chemical compositions are difficult to measure. They are estimated and sometimes based on operator heuristics. Also the mathematical models are based on parameters which are not well known and changing with time.

There are long time constants, like thermal conditions and hot metal balance, blast volume, furnace heat index, as well as very short time constants like blast furnace pressure. That is, it is a multivariable, multiprocess, multirate system.

Although the common applications refer to diagnosis and monitoring, some RT applications have been already reported, [12].    In [13], an interesting combination of classical and AI techniques is presented: physical models are used to forecast the operating conditions and to optimize the furnace behavior in parallel with fuzzy controllers copying with deviations.

Also, in [14], NN to model the BOF behavior and to predict the finishing time and other characteristics are applied.

Soaking Pits.

In order to prepare the ingots with a uniform temperature before rolling them, they are introduced in batch reheating furnaces, where the saving of energy and the on-time operation are basic in the whole steel making process.

Again, difficulties in measurement and uncertainties in the parameters are the key reasons to use AI techniques, [15].

Hot Strip Finishing Mill

This is a paradigmatic example of complex system where hierarchical control has been applied, [16]. A number of variables, strip gauge, looper angle, and strip tension, are simultaneously controlled and jointly optimized to reach a global objective. Instead of simple decentralized control, interactive control has been widely used, (see, for instance, [17]).

The high nonlinearities of this process leads to the use of NN as a complementary tool to model and implement classical control strategies. In [18], NN are used for both, off-line modeling of the stands, and on-line adaptive tuning of the parameters of the algorithmic model or adding corrective actions.

###### Integrated Controllers

As previously described, in a large and complex industrial process there are a variety of tasks that must be performed to keep the system under control. Although based on different data sources, a number of manipulated variables (inputs to the actuators) should be computed. All these tasks impose different requirements to the execution environment, but from the point of view of the control system designer, it is very useful to have a unified approach to reduce the workload in the development process, [19].

The main requirements are:

	The control system can work in several modes to solve different control situations with unlike goals, such as normal operation, start-up, shut-down, failure, etc. For each mode, different actions must be performed that can be solved with any of the techniques outlined above.

	The tasks can be executed with a periodic scheme or can be launched when an event is detected in the control system (event driven).

	The development system must hold different techniques (AI or conventional ones) at the same time, so it is necessary to dispose the appropriated algorithms and data structures.

	For large systems it is desirable to distribute the computational load between various CPUs to increment the execution speed.

Based on these considerations, it is useful to have an architecture able to support different types of techniques for implementing control algorithms matching the RT constraints. This architecture is based on the idea that any of its components, can be seen as an information process that produces a conclusion or output from an environment data set or another source. This base component will be named Intelligent Regulator Agent (IRA), [20] because this module is part of a system that is used to control or regulate a process and it can support algorithms derived from artificial intelligence techniques or conventional ones. Each IRA, as shown in fig. 4, defines a set of problem solvers (rule-based, algorithmic, genetic, etc.) in order to provide solution to a specific    subsystem. Also, they are structured for achieving the real time constraints imposed to the control system.

Various components can be considered in an IRA:

Regulator knowledge: It defines the functionality of the IRA by means of the declaration of the structures, parameters or information needed to characterize its behavior. This information is dependent on the technique used in the regulator (real valued parameters for linear discrete regulators, production rules in expert systems, topology and learning methods for neural networks, etc). Also, various levels can be defined to cope with the real time constraints. Each level produces a better response of the system, but it takes more time to compute the final solution, so the scheduler must select the level that produces the best response in the CPU time assigned to evaluate the IRA.

Local data base: A local area to store some temporary and local results produced along the evaluation of each agent is necessary. When the evaluation ends, all or some data is transferred to the global data base and can be shared by another modules.

Evaluation agent: This element implements the chosen algorithm and obtains a result. It depends on the used technique and it is necessary to have a library of different methods that are selected on the design stage.

The scheme of the agent activity depends on the algorithm implemented in the evaluation agent. If a procedural regulator is used (discrete linear controller, neural network or fuzzy controller) then the required execution time is bounded and can be known, so an a priori scheduling can be used and there is no optional part. Moreover, if an anytime algorithm is used then the task is executed until the maximum executed time is elapsed and the current solution is exported to the global data base. In both cases only one level of the regulator knowledge is used.

Regulator

Knowledge

Evaluation agent

Local

Data Base

Version 1

2

3

4

Outputs

inputs

Figure 4. Intelligent regulator agent structure

But if the execution time of the algorithm is not bounded or it may have very large variations depending on the input data presented to the system, different levels or versions must be used in order to allow that a response can be available on-time, even with less accuracy. It is like a discrete anytime algorithm: it improves the response when a level is executed, then the next one is taken and the response is again improved until the execution time is expired and the last response is concluded.

Examples of this approach can be found in expert system for the direct control or supervision of a process. The first level or version of the intelligent regulator should be a system with bounded execution time like a fuzzy controller. It can be modeled like one layer production rules (relates directly the input variables to the output ones), or with general production rules without using computing intensive functions in the antecedents of the rules (like temporal logic functions). The next level should improve the answer taking into account more variables and information. Then, upper levels can include first order predicate calculus and temporal information. The optional tasks of this agent will be the tasks formed at level 2 or higher.

For the design of the global controller it is necessary to start with a decomposition on specialized agents [21] that focuses in different problems or situations that can be met in the process. These agents can be independent or some of them can use information derived from another one. For every agent a technique must be selected.

###### Hierarchical Control

The main characteristic of the hierarchical schemes is the fact that the final control action on the process is obtained through a chained sequence of intermediate control actions calcula. This is different from the progressive action refinement where successive controllers are run, each one refining the control action provided by the previous one.

In a hierarchical control structure there are different control actions belonging to different levels of decision in the system. Therefore, there is a hierarchy of controllers corresponding to a number of fixed levels. Usually the controllers at the highest levels are AI-based, while the ones at the lowest are mainly based on classical control theory.

The AI-based highest levels in the hierarchy carry out tasks requiring an important degree of reasoning, such as diagnosis and supervision and, in them, progressive refinement techniques may be also possible.

Quality control is one of the cases in which this kind of hierarchical schemes is often encountered. Typically, quality defaults are detected and classified (for instance, by means of a NN-based pattern classification). Depending on the detected defaults and other relevant variables of the process, an inference mechanism on a rule-based system yields new values for the set-points of some process variables. In turn, some lower level controllers take these set-points and manipulate the appropriate process variables so as to keep them within given specifications.

Regarding the control implementation, each of the aforementioned levels can be implemented as a program plan, that is a set of tasks to be executed. An example of the structure of a typical plan is depicted in figure 5, where a plan for production scheduling in an industrial application is shown.

Fig. 5. Production plan

Each of the boxes represents a task to be executed if some conditions are satisfied. The plan is periodically activated. The whole plan can be programmed on a single program thread, or several ones can be used if some of the tasks in the plan can be concurrenly executed.

On the other hand, the plans defined in the system architecture interact among them, as depicted in fig. 6, where a blackboard is used to keep and share information.

Fig. 6. Plans interaction

From the point of view of the time requirements, two levels can be considered:

- Plans architecture. As said before, in an industrial complex control system, an ordered hierarchy of control functionalities allows for managing different decision levels. Each of these levels is implemented as a set of tasks, so called a plan. Different functioning modes, either set by external events to the plan or by internal ones, may switch on and off some of the facilities of the control system. This introduces optionality among tasks, therefore modifying the available processor time for other tasks. For the number of tasks is finite, there is a finite set of options. For each one of them all the tasks involved must be executed, being the time required known and fixed if it is so for each of the tasks in the plans. This is different from the optionality appearing in progressive refinement, where the optional part duration is not fixed, but depends on the processor time availability. Therefore, optionality among tasks consists of the selection of a `program line' out of a finite, known, set of them. The swapping between `program lines' is sporadic.

- Tasks implementation. As previously mentioned, each of the tasks within a plan may be composed of three parts compulsory, optional and final.

In many cases the high level reasoning takes place with a long time period or even sporadically, while the lower levels are activated with short periods. That is, several time scales may be needed.

RT implementation of optional tasks

If an AI technique is used, the way the answer is obtained is relevant. In this sense, two different approaches can be considered:

The same technique or algorithm is applied and depending on the available time the result has a higher quality. Anytime algorithms are one of the main representative technique in this approach. In this kind of algorithm, the computation scheme can be described as follows: a mandatory part with a required computation time known is executed first, followed by a improvement part (optional part) which has the form of a classical iterative algorithm, finishing with a final part which send the output to actuators. The optional part can be interrupted at any time and the solution correspond to the last finished iteration. This behavior is shown in fig. 7 and table I.

Fig. 7 Anytime option

Table 1. Optional tasks

Optional part (block 2) is implemented as a control structure provided by the language Ada95. This Asynchronous Transfer Control structure allows the execution of a piece of code limited by the time specified in the first part of the select.

Another approach consists in the definition of different pieces of to solve the same problem. Each piece uses a different model of the problem (more or less detailed) and, as consequence, it requires more or less time. Depending on the available time, the richest approach will be selected. As before, there is a mandatory part providing a basic solution and a number of possible options is ready to be executed, as shown in fig. 8.

Fig. 8. Selected option

The inclusion of optional parts in control system algorithms allows the use of the remaining CPU time to improve the response to some control demands. There is always some remaining CPU time because some sporadic activities related to alarms, threshold detection and emergencies are seldom executed, and for any task, the a priori scheduler assumes more time than usually required to prevent the worst case, that is, the occurrence of blocking of resources, full data, and so on. The saved time that can be invested in the improvement of some results.

The execution of optional parts will be determined by the availability of remaining time and the demand for soft sporadic load. Optional parts are not executed at the same priority level then that of the mandatory ones. Several approaches to include optional computation in real time systems can be found in the literature. In one of the first works in this line [22] a task model to incorporate imprecise computation was defined.

From the scheduling point of view, optional parts should be handled in a specific way:

- they should be executed after the completion of their related mandatory part

- they should be selected based on their priority, required execution time, and time availability before the deadline of their related task.

- they should be enabled if the data they require from other tasks is available in the common memory (blackboard).

### Optional server

All optional components can be included at run-time through a server process (the optional server). Initially, the optional server component does not affect this off-line feasibility, as it is always executed in slack time. However, at run-time some action must be taken if the guaranteed deadlines are affected [23].

In [24] several strategies have been defined to execute optional parts. The optional server executing at the higher priority of the system with a policy based on task importance is shown in fig. 9, where two concurrent tasks are considered. In this case, task 1 has the higher importance on the system and, as a consequence, when there is available time and need of optional computation, the optional part is executed even delaying the execution of task T2 (without breaking the    deadline).

Fig. 9. Optional server using an importance criteria

These strategies have been evaluated and compared. The criteria used for each are:

- based on the user importance. The user importance does not coincide with the system priority.

- sharing the optional part of all tasks to equilibrate the result improvement

- applying the second strategy but selecting the best fitted optional part.

The main conclusion of this evaluation is the difficulty to choose a strategy because in most cases the best solution depends on the task process and the solution is problem dependent. Thus, more than magic general solutions, a trial and error approach with off-line previous simulation will determine the best structure for a given application or operating scenario.

###### Applications

As previously mentioned, RT applications of AI in the process industry are becoming more and more common. Our department has been involved in the development of two typical applications in the control of kilns, one of the basic subsystems in the process industry. They illustrate the real-life application of the control structures proposed above.

Ceramic Tile Production

Like in the steel industry, ceramic tile production critical variables are related to temperatures and compositions which are, many times, difficult to on-line measure. Thus, the production is critically dependent on the firing process at the kiln. Current control systems only apply local control, and product quality is not directly considered. As consequence, there are important material losses and process inefficiency. MARCK (Multi-process Adaptive Real-time Control for a Kiln), is a tool developed under the ESPRIT program aiming at the application of high technology in the production control of quality tiles.

The general architecture of MARCK is depicted in fig. 10. It consists of a set of program threads sharing information through blackboards, and driven by events, [25].

Figure 10: Global threads architecture.

The kiln control devices are the PIDs driving the burners, the PLCs managing alarms and auxiliary devices, and the set of sensors, in particular, the planar and caliber measuring system. They communicate with the application through the coordinating and the quality control sets of tasks (described below), while the kiln operator does through the man-machine interface.

All the current data is stored in a set of blackboards, and the communication among the application tasks is carried out through shared blackboards. Long-term data regarding the production is stored in a database, which also holds all the information about the different tile models.

From the control point of view, the main functionalities sought are:

- Control of the kiln temperature profile.

- Adjustment of the kiln to lack of entering material (voids).

- Transfer between temperature profiles (tile model).

- Quality control, with main care taken on dimensional problems.

For this purpose, the devised hierarchical control architecture consists of three levels:

- Low level control: at this level, the local PID controllers at each zone along the kiln drive the associated burners.

- Coordinating control level: this level uses a dynamic model of the kiln to take into account the interactions among the different kiln zones. Accordingly, the set-points sent to the PID local controllers    are modified using a decoupling adaptive feedforward control strategy. Due to the uncertainty of the interaction parameters, an adaptive fuzzy decoupler is well suited.

- High, rule-based control level : defines high level set-points for the temperature and pressure profiles along the kiln, with the aim of counteracting quality defects appearing in the tiles. Different strategies can be defined by the process engineer, attending to the defect to be controlled and the kind of material.

A basic scheme of the proposed control architecture is depicted in fig. 11, where the local PID controllers are assumed to be constitutive part of the kiln.

Figure11: Global control scheme.

Different operation modes in the application allow for switching on and off the control levels:

- Monitor: read-only mode at which no control action is taken.

- Manual: local PID set-points are directly provided by the kiln operator through the man-machine interface.

- Automatic low: only the coordinating control is carried out.

- Automatic high: both coordinating and quality controls are switched on.

The goal of the decoupling controllers is to achieve a temperature behavior only defined by the local PID settings at each section, independently from the neighbor.

The coordinating control is implemented as a    periodic set of tasks, being the responsible for acquiring data from the kiln through the PIDs, and writing on them.

At the quality control level the main goal is to generate new set-points for the controlled kiln variables so that the quality defects at the output tiles can be corrected. MARCK treats a quality default trying to emulate the steps followed by a human operator when faced with the same problem:

- Select the kiln sections where an action should be applied. This is referred as the strategy selection.

- Calculate, by means of a fuzzy rule base inference, the temperature modification to be applied in the selected sections. This modification is considered on the temperature difference between the upper and bottom profiles.

- Distribute the calculated gradient modification between the upper and bottom temperature profiles.

- Correct the temperature set-points of left and right neighbor sections if the temperature gradient is too large (sympathy action).

Cement Kiln Control: Rigas

A real time expert system (RIGAS), [26-28], has been developed and applied to the control of a cement kiln. RIGAS allows the definition of Intelligent Regulator Agents, and different control schemes can be implemented. The control system can work in several modes to solve different situations (normal, alarm, heating, etc.) and several levels of solution can be defined in each regulator to cope with the real time requirements.

The main characteristics of the kiln process are : distributed process ; different operation situations; large number of variables ; measuring delays ; heuristic used in the control. These properties justify the use of AI techniques in the control of this system and also real time requirements must be imposed because the direct interaction between the system and the process.

The kiln operation situations can be classified in regular (start up, shutting down, normal production, etc.) and exceptional (coolers obstruction, raw cut off, cyclone obstruction, etc.). All them require a different control policy and this implies that diverse controllers must be used. The control system is organized in three blocks modeling the basic situations : secure operation, when the control goal is the production ; exception operation, which require a set of specific actions outside normal functioning and transient operation, that includes the actions needed to reach the secure operation. The developed system uses an IRA to identify the current situation and the transitions between them, activating the specific blocks designed for that situation. This structure can be seen in figure12.

Normal

mode

Secure operation

Start up

mode

Cooler

obstruction

Transient operation

Exception operation

Fig. 12 : Kiln situation modes

In order to show the methodology to implement the control system, the normal mode structure is presented in fig. 13. In this mode, the control goal is to reach maximum production with the minimum coal consumption. The kiln state is divided into three static states (hot, normal, cold) and three dynamic ones (heating, stable or cooling). An IRA is used to conclude the kiln state and a confidence coefficient from the measured values. Next, this information is used in 4 IRA, one for each output variable (coal flow, exhaust aperture, kiln revolutions and raw flow), to obtain the numeric values of the respective control action that is applied to the process.

IRA

Kiln status

IRA

Coal

IRA

Exhaust

IRA

Rpm

IRA

Raw

Input

data

Kiln status

Fig. 13. Kiln normal control mode.

At the first level of each IRA, a fuzzy controller with only one level of rules (the rule antecedent are input variables and the consequent are output variables) has been implemented. These rules model the heuristic behavior of the kiln operators.    At the second level, more information is processed, like temporal response of the variables or reasoning with qualitative models. Finally, at the third level an optimization algorithm based on an experimental model is implemented.

###### Conclusions

The appealing properties and friendly use of some AI techniques promote their use even at the control levels close to the process, where the time constraints are usually hard. Among these constraints are the sampling period, the delay between sampling and updating the control action, the variation of this interval, and the synchronism for data exchange between processes. Some of them are strongly influenced by the computation of the control action by softcomputing techniques, because there is always some optionality and this should be taken into account in the design of the control system.

In order to get some insight in these problems, a review of timing issues from both viewpoints, control and software implementation, has been made.

To use conventional and AI control techniques in a complementary way, two main strategies have been proposed. The intelligent regulator agents are organized in such a way that to compute their output, the control action, some options are possible. If there is a minimum time to execute each one of them, the one to be used will be determined each time by the CPU time availability. These integrated controllers keep all the information they need to run the different control options. This solution has been successfully applied to control the cement kiln in a cement factory.

A classical control approach, the hierarchical control, has been also proved to be suitable to combine both techniques. In order to allow an easy data exchange between tasks, local and global blackboards are implemented in connection with the different control activities. Although AI techniques are more appropriated for higher levels in the control hierarchy, they can be easily implemented for intermediate and lower levels of control. This hierarchical structure allows a better control of the execution time by means of a modular computation. This strategy has been applied in the control of a ceramic kiln where the operating scenario often changes.

#### References

[1] Narendra, K. Adaptive Control using Neural Networks. In Neural Networks for Control, MIT Press, 1990.

[2] Antsaklis P.J. Neural Networks in Control Systems. IEEE Control Systems, 10(3), Apr. 1990.

[3] Verbruggen H.B. and Krijgsman, A.J. and Bruijin, P.M. Towards Intelligent Control : Integration of AI in Control. In Application of Artificial Intelligence in Process Control, Pergamon Press, 1992.

[4] Lu, Y-Z. Industrial Intelligent Control. John Wiley and Sons. 1996.

[5] Albertos, P.    and Crespo, A. Real-Time Control of Unconventionally Sampled Data Systems. IFAC Workshop on AARTC´97. Vilamoura, pp 1-13. 1997.

[6] Audsley N.C. "The Deadline monotonic scheduling". Real-Time Systems Research Group. Department of Computer Science. University of York, UK. Report number YCS146. 1991

[7] Narendra, K. And Mukhopahyay, S. Intelligent Control using Neural Networks. Procs ACC, pp.1069-1074, Boston, MA, 1991.

[8] Ljung, L. and Sjoberg ,J. A System Identification Perspective on Neural Nets. Tech. Report, Dept. Of Electrical Eng., Linkoping U., Sweden, 1992.

[9] Albertos, P.    Fuzzy Logic Modeling and Control. In Identification Adaptation, Learning. S. Bittanti and G. Picci (Ed.) Springer pp 479-513. 1996.

[10] Picó, J. and Moya, P. Localized BF-type Networks for Identification and Adaptive Control of Discrete.time Nonlinear Systems, Proc. IFAC Symp SICICA’97, Annecy (France), 1997.

[11] Wang, L.-X. Adaptive Fuzzy Systems and Control, Prentice-Hall, 1994.

[12] Lu, Y.Z. 1996a. “Meeting the Challenge of intelligent system technologies in the iron and steel industry”. Iron and Steel Engineer. September 1996. pp 139-148.

[13] Iida, O., Ushijima, Y and Sawada, T. Application of AI techniques to blast furnace operations. Iron and Steel engineer. pp 24-28. October 1995.

[14] Yun, S. Y. Chang, K.S. and Buyn, S. M. Dynamic Prediction using NN for Automation of BOF Process in Steel Industry. ISSM Aug. 1996. pp 37-42.

[15] Lu, Y-Z. And Williams, T.J.Computer control strategies ... for the control of steel Mill Soaking Pits. ISS Trans. Vol. 2 pp 35-43. 1983

[16] Singh, M. G. Dynamical Hierarchical Control. North Holland. 1980.

[17] Okada, M. et al . Optimal Control System for Hot Strip Finishing MillSession 7b-04, pp 493-498 IFAC World Congress    1996.

[18] Portman, N.F. Lindhoff, D. Sorgel, G. and Granckow O.    . Applications    of NN in rolling mill automation. Iron and Steel Eng.  Pp 33-36, Feb 1995.

[19] Morant, F., P. Albertos, J.L. Navarro, and A. Crespo, Intelligent regulators: Design and Evaluation. Proc. 12th IFAC World Congress, Vol. 7, pp. 479-484, Sydney 1993.

[20] Albertos, P. A. Crespo, F. Morant, and J.L. Navarro.    Intelligent Controllers Issues. Proc. IFAC Symp. SICICA'92, pp. 321-334, 1992.

[21] A. Crespo, J.L. Navarro, R. Vivó, A. García, A. Espinosa, A real time environment for process control. Proc. Int. Workshop on Artificial Intelligence in Real Time Control,1991.

[22] Burns, A. (1993). "Preemptive Priority Based Scheduling:    an Appropriate Engineering Approach" Real-Time Systems Research Group. Department of Computer Science. University of York, UK. Report number YCS214.

[23] Audsley, N.C., Burns, A., Richardson M.F. and Wellings, A.J. (1991). "Incorporating Unbounded Algorithms Into Predictable Real-Time Systems". Department of Computer Science. University of York, UK. Report number RTRG/91/102.

[24] Hassan H., Crespo A. "Scheduling Intelligent Tasks with Optional Parts in a Real-Time Environment". 3th IFAC/IFIP Workshop on Algorithms and Architectures for Real-Time Control. May-95.

[25] Bondia, J, Moya, P. Picó, J. and Albertos, P. MARCK: An Intelligent Adaptive Real-Time System for Ceramic Kiln Control. Interfaces 97. Montpellier. May 1997.

[26] A. Crespo, J.L. Navarro, R. Vivó, A. García, RIGAS: an expert server task in real-time environments. Proc. Int. Symp. on Art. Intel. In Real-Time Control, pp. 631-636, Delft, 1992.

[27] Navarro, J.L., P. Albertos, F. Morant, M. Martinez, and R. Vivó, The Cement kiln: AI Approach to Model and Control, IFAC Int. Symp. ADCHEM'91, pp.331-334, Toulouse, 1991.