DINAMYC

INTERCONNECTION NETWORK

Shared path network

Single bus

Multiple buses

Switching network

Crossbar

Multistage network

Dynamic interconnection networks

enable the temporary connection of any two components of a multiprocessor.

There are two main classes according to their working mode:

Shared path networks - those networks that provide continuous connection among the processors and memory blocks

It was typically a single bus in the first generation multiprocessors. In recent third generation machines hierarchical bus-systems are introduced.

Switching network - does not provide a continuous connection among the processors and memory blocks, rather a switching mechanism enables to temporarily connect processors to memory blocks.

Drawbacks: - too expensive (switching networks)

- they can support only a limited number of processors (bus connection)

they

Shared path networks

Single shared bus

Advantages:

- Its organisation is simply a generalisation and extension of the buses employed in uniprocessor systems.

- It contains the same bus lines (address, data, control, interrupt) as uniprocessors and some additional ones to solve the    contention on the bus when several processor simultaneously want to use the shared bus.

These lines are called arbitration lines

- It is very cost-effective interconnection scheme.

Drawback:

The contention on the shared bus represents a strong limitation concerning the number of applicable processors.

The typical structure of a single bus based multiprocessor without coherent caches is shown below:

address

data

control

interrupt

bus exchanges

lines

...

...

...

I/O m

I/O

1

Mn

M1

Pk

P1

Bus arbiter and control logic

Comparison of write latencies of various buses

The uniprocessor and multiprocessor buses are very similar, there is a significant difference in their working mode.

1. Uniprocessors and first generation multiprocessors employ:

- locked buses (like the Multibus, VMEbus, etc.)

2. Second generation multiprocessors apply:

- pended buses.

The difference comes from the way how memory accesses are handled on the bus. A memory write access needs two phases:

Phase 1. Transferring address and data on the bus to the memory controller.

Phase 2. Executing the memory write operation (including parity check, error correction, etc.) by the memory controller.

In a uniprocessor system the two phases are executed sequentially, the bus is locked until the complete memory operation is finished.

( figure (a))

In single bus based multiprocessors after finishing the bus transfer phase for a memory unit, another processor can start a new write operation for another memory unit.

Bus transfers and memory controller operations can be overlapped.

The execution time can be radically reduced by a pended bus provided that the memory operation is three times longer than the bus transfer time (figure (b)).

P4

P3

P2

P1

processors

time

481216

(a) Memory write on locked bus

memory cycle

bus cycle

P4

P3

P2

P1

processors

time

(b) Memory write on pended and split transaction buses

47

A memory read access needs three phases:

Phase 1. Transferring address on the bus to the memory controller.

Phase 2. Executing the memory read operation by the memory controller.

Phase 3. Transferring data on the bus to the requester processor.

The three phases must be executed sequentially ( figure a).

In a multiprocessor system:

Phase 1 does not require the use of the data bus, while phase 3 can be executed without the use of the address bus. Hence combining the first phase and the third phase of two memory reads, executed on different memory units, reduces the effective phase number to two( figure b).

Requirements of pended bus:

- A./ Memory controllers should contain an input buffer where the contents of the address and data bus can be temporarily stored from the bus, and an output buffer where the result of the memory read can be stored until the bus accepts the memory request to transfer the data.

- B./ The address and data buses should be separated in order to combine the first and third stages of memory reads.

- C./ The memory controllers should be able to request bus transactions and arbiter logics should be able to service their requests.

Third generation buses

They enable the requester processor to issue a new memory read access before completing the previous one. Data from different memory reads can be returned in arbitrary order.

The first phase and the third phase of a memory read is totally separated or split and hence these buses are called split-transaction buses. (figure c).

Using optimising arbiter logics two bus transaction phases (phase 1 and phase 3 of different memory reads) can be executed simultaneously.

(a) Memory read on locked bus

(b) Memory read on pended bus

(c) Memory read on split transaction bus

Comparison of read latencies of various buses

Arbiter logics play a crucial role in the implementation of pended and split-transaction buses. These are the so-called 1-of-N arbiters since they grant the requested resource (the shared bus) only to one of the requesters.

There are two main possibilities to organise the arbitration logic according to the distribution of its components in the multiprocessor system.

Bus allocation policies belong to one of the five schemes.

Bus requests can be sent to the arbiter either by a single shared request line or by independent request lines.

Granted requester can be notified either by one of the independent grant lines or by using a daisy-chained grant line.

ARBITER LOGICS

Organisation

Distri-

buted

Handling of requests

Shared requests

Independent requests

Centralised

Bus allocation policy

fixed

priority

least recently used

rotating

round robin

first come first served

Handling of grants

Daisy chained

Independent grants

Centralized arbitration with independent requests and grants

1. Masteri requests the bus by activating its dedicated bus request line.

2. If the bus busy line is passive, i.e., no other master uses the bus, the arbiter immediately allocates the bus to the requester by activating the granti line. The requester deactivates its request line and activates the bus busy line prohibiting the allocation of the bus for other requesters. After completing the bus transaction, the requester deactivates the bus busy line.

3. When the bus busy line is active, the arbiter does not accept any bus requests.

4. When several request lines are active by the time the bus busy line becomes passive, the arbiter can use any bus allocation policies described above.

Advantages:

- it can realise any bus allocation policy

- it has faster arbitration time compared with the daisy-chained arbiters

Drawback:

-.it requires a large number of bus exchange lines

Daisy-chained bus arbitration scheme

/centralised version with fixed priority policy/

1. There is only a single shared bus request line in the daisy-chained bus arbitration scheme. All the masters use this line to indicate their need to access the shared bus.

2. The arbiter passes the bus grant line to the first master and then it is passed from master to master creating a chain of the masters. The priority of a master is determined by its position in the grant chain. The closer position to the arbiter represents a higher priority.

3. A master can access the shared bus if the bus busy line is passive and its input grant line is active. When the master does not require the bus and receives an active grant line, it activates its output grant line enabling the next master to use the bus.

Advantage:

- very cost-effective

Drawback:

- slow propagation of the grant signal on the grant chain

Structure of a decentralized rotating arbiter with independent requests and grants

The priority loop of the rotating arbiter works similarly to the grant chain of the daisy-chained arbiter.

1. Arbiteri is allowed to grant its coupled master unit if masteri has activated its bus request line, the bus busy line is passive and the priority(i-1) input line is active.

2. If masteri has not activated its bus request line, arbiteri activates its output priorityi line.

The main difference comparing to the daisy-chained scheme:

- in the daisy-chained arbiter the lowest priority unit is always the farthest master,

- in the rotating scheme, it is always the master that releases the bus.

Multiple shared bus

Problem: the limited bandwidth of the single shared bus

Solve: => to multiply the number of employed buses similarly to the processors and memory units.

Four different ways:

1. 1-dimension multiple bus system

2. 2- or 3-dimension bus systems

3. cluster bus system

4. hierarchical bus system

1-dimension multiple bus system / the simplest generalisation/

This approach leads to a typical uniform memory access (UMA) machine where any processor can access any memory unit through any of the buses.

Structure of a 1-dimension multiple bus multiprocessor

The arbitration is a two stage-process in 1-dimension multiple bus systems:

A./ The 1-of-N arbiters (one per memory unit) can resolve the conflict when several processors require exclusive access to the same shared memory unit.

B./ After the first stage m (out of n) processors can obtain access to one of the memory units.

When the number of buses (b) is less than that of the memory units (m), a second stage of arbitration is needed where an additional b-of-m arbiter is employed to allocate buses to those processors that successfully obtained access to a memory unit.

Structure of a B-of-M arbiter:

/a state register and m arbiter modules (A1, A2, ... , Am) for each memory unit/

/State register: - round-robin arbitration policy/

2-dimension multiple buses - in the Aquarius Multi-Multi architecture

3-dimension multiple bus - in the Wisconsin Multicube machine

In these systems, multiple buses compose a grid interconnection network. Each processor node is connected to a row bus and to a column bus. Processors along a row or column constitute a conventional single bus based multiprocessor.

Memory distribution:

- memory units attach to each bus /most traditional/

- only the column buses are supplied with memory units / in the Wisconsin Multicube machine/

- the memory units are allocated to processor nodes rather than to buses / in the Aquarius Multi-Multi architecture

The main problem : the maintenance of cache coherency.

cluster BUS SYSTEM

It represents a NUMA machine concept.

The main idea: single bus based multiprocessors, called clusters, are connected by a higher level bus. Each cluster has its own local memory.

Access time of a local cluster memory << Access time of a remote cluster memory.

The code and stacks in the cluster memory Þ reduce the need of accessing remote cluster memory.

The GigaMax cluster architecture:

Switching networks

Crossbar

Advantages:

- most powerful network type

- it provides simultaneous access among all the inputs and outputs of the network providing that all the requested outputs are different.

- the large number of individual switches which are associated with any pair of input and output of the network

Drawback:

- enormous price

- the wiring and the logic complexity increase

Schematic view of a crossbar network:

Detailed structure of a crossbar network

All the switches should contain:

- an arbiter logic to allocate the memory block in the case of conflicting requests

- a multiplexer module to enable the connection between the buses of the winner processor and the memory buses.

Multistage networks

This is a compromise between the single bus and the crossbar switch interconnections

/from the point of view of implementation complexity, cost, connectivity, and bandwidth/.

A multistage network consists of alternating stages of links and switches.

They can be categorised based on the number of stages, the number of switches at a stage, the topology of links connecting subsequent stages, and the type of switches employed at the stages.

The complete design space of multistage networks:

Omega network

This is the simplest multistage network:

- It has log2N stages with N/2 switches at each stage.

- All the switches has two input and two output links.

- Any single input can be connected to any output.

- Four different switch positions:

upper broadcast,

lower broadcast,

straight through,

switch.

Omega network /2

These networks are suitable for broadcasting information from a single input to all the outputs due to the upper broadcast and lower broadcast switch positions.

The state of the switches when P2 sends a broadcast message.

Blocking network

-Any output can be accessed from any input by setting the switches, but:

-the simultaneous access of all the outputs from different inputs is not always possible.

The possible sets of transformations mapping all inputs to a different output-=> permutations.

In blocking networks there are permutations that can not be realised by any program of the switches.

Blocking in an Omega network:

No matter how the other inputs are mapped to the outputs, a conflict appears at switch A, resulting the blocking of either 0->5 or the 6->4 message.

A

Blocking and nonblocking network

Blocking networks: - multistage networks

The simultaneous access of all the outputs from different inputs is not always possible.

Possibility of improvement of the parallel access mechanism:

- additional stages to introduce redundant paths in the interconnection scheme /Benes network/ => rearrangeable nonblocking network.

/=> increased size, latency, and cost of the network/

Multistage networks were quite popular in early large-scale shared memory systems /for example: NYU Ultracomputer, CEDAR, HEP, etc./

Nonblocking network: - crossbar interconnection

Any simultaneous input-output combination is possible.

The summary of properties of multistage networks:

| network type | number of stages | number of switches at a stage | topology of links between stages | switch size | operation mode |
| --- | --- | --- | --- | --- | --- |
| omega | log2N | N/2 | 2-way shuffle | 2x2 | blocking |
| butterfly | log8N | N/8 | 8-way shuffle | 8x8 | blocking |
| genera-lised- cube | S=log2N | N/2 | [0,1]: shuffle [1,S]: exchange | 2x2 | blocking |
| Benes | S=2*log2N-1 | N/2 | [2,S-1]: exchange | 2x2 | rearrangeable nonblocking |

Techniques to avoid hot spots

In multistage network based shared memory systems hundreds of processors can compete for the same memory location. This place of the memory: => hot spot .

Problem:

They enter at two different inputs to the switch but want to exit at the same output.

Solutions:

queuing networks

These temporarily hold the second message in the switch applying a queue store being able to accommodate a small number of messages.

nonqueuing networks

These reject the second message so that unsuccessful messages retreat and leave the network free.

Hot spot saturation in a blocking Omega network:

Techniques to avoid hot spots /2

Solutions /cont./:

combining networks

Introduction of combining switches: are able to recognise that two messages are directed to the same memory module and in such cases they can combine the two messages into a single one.

This technique is particularly advantageous in the implementation of synchronisation tools like semaphores and barriers which are frequently accessed by many processes running on distinct processors.

This structure used in the NYU Ultracomputer (shown below).

- If the two requests refer to the same memory address

Þ the corresponding combining queue forwards one request to the memory block

Þ places the second request in the associated wait buffer.

Structure of a combining switch:

Cache Coherence Problem

Data Categories:

Coarse-grain units of information processing are processes.

Types of data causing less or more problem with coherence:

	read-only: no change in run-time (program-code, constants) è no change - no problem with coherence

	private writable/readable: used by a single process (local variables, process state variables) è problem only in case of process migration

	private writable / shared readable: a single process manages all changes, but more processes read the result è problematic

	shared writable/readable: used (and written) by more processes (global variables) è the most problematic

These types of data can be separated by compiler (and/or user) assistance.

Levels of solution:

	HW-based protocol for all data categories - total but complex solution

	SW-based solutions with some HW support, restrictions - compromise

	Shared writable data are not cached - compromise, no solution for critical situations

HW-Based Cache Coherence Protocols

We discuss hardware-based protocols from three points of view:

	How they keep coherence of the updated local copies and the 	main memory

è memory update policy

	How they keep coherence of several local copies

è cache coherence policy

	How they work in detail (algorithm and data structures)

è protocol type,

what is determined mainly by the interconnection network

Memory Update Policies

write-through: (a greedy policy)

	as a data is updated in one of the local caches, its copy in the 	main memory is immediately updated, too

	unnecessary traffic on interconnection in case of private data and 	of infrequently used shared data

+	more reliable (error detection and recovery features of the main 	memory)

write-back: (a lazy policy)

	data in memory is updated only at some events (eg data is 	replaced or invalidated in the cache)

	allows a temporary incoherence of caches and memory

	while not updated, read-references to memory will be redirected 	to the appropriate cache

	more complex cache controllers

Memory update policies in writingCache coherence policies in writing

shared data structuresshared data structures

Cache Coherence Policies

write-update: (a greedy policy)

	as data is updated in one of the local caches, its copies in other 	caches immediately updated , too

	copy in the main memory may or may not be updated

	immediate data migration, unnecessary traffic in case of private 	data and of infrequently used shared data

	cache controllers have to accept requests not only from their own 	processor, but from other cache controllers

write-invalidate: (a lazy policy)

	as data is updated in one of the local caches, all other copies in 	other caches and in the main memory are immediately invalidated

	while not updated, data is provided by the updating processor's 	cache for read operations of other processors

	cache controllers have to accept invalidate command from other 	cache controllers

Protocol Types

snoopy cache protocol: (used mainly in single bus interconnections)

	both updating and invalidating versions are used

	updating processor broadcasts the update or invalidate command to all other caches

	cache controllers 'snoop' on the bus for coherence commands, and update or invalidate their cached blocks if necessary

hierarchical cache protocol: (used in hierarchical bus interconnections)

	according to the hierarchical structure, starting from the bottom level, we can place a 'supercache' at each segment of the bus, which supercache serves as a connection to the higher level bus

directory schemes: (used in general interconnections)

	updating processor multicasts coherence commands exactly to those caches having a copy of the data

	several directory schemes

Scaleable Coherent Interface: (an interface definition to interconnection)

	IEEE 1596-1992 standard, distributed directory, point-to point model

Snoopy Cache Protocols

Basic solution

	Memory always up-to-date (write-through) and

	Write-invalidate

Situations:

at references of the local processor:

Read hit: use the copy from local cache (no bus-cycle)

Read miss: fetch from memory (cache replacement policy) (bus-cycle)

Write hit: invalidate other caches, update cache and memory (bus-cycle)

Write miss: fetch from memory (cache replacement policy), invalidate other caches, update cache and memory (bus-cycle)

Replacement: find free or chose victime, nothing to do with the old content, load new block from the memory

at bus cycles of other processors:

Recognizing a write-cycle on the bus - executed by an other 	processor - to a block, a valid copy of which the local cache has, the 	local copy of the block should be invalidated.

Definition of commands to be performed at various read hit/miss actions:

a. Read hit:

The read operation can be performed locally without any bus interaction.

b. Read miss:

I. There are shared copies of the data block (indicated by an extra bus line called the "shared line"):

Þthe containing caches deliver the required copy (see figure a.) where Pj has a read miss.

II. A dirty copy exists:

Þit supplies the necessary copy

Þupdates the main memory.

All the copies become shared (see figure b.)

III. There is no cached copy of the block:

Þthe memory sends the block which becomes valid-exclusive.

Definition of commands to be performed at various write hit/miss actions

c. Write hit:

I. The state of the block is valid-exclusive or dirty:

Þthe write operation can be performed locally

Þthe new state will be dirty.

II. The block is shared:

Þall copies, including the memory one, will be updated (write-update, write-through policy).

If sharing has ceased, the block becomes valid-exclusive.

d. Write miss:

I. Only the memory contains the block:

Þit should supply a copy which will be updated at the requester processor

Þafter that it becomes dirty (see figure a).

II. Shared copies are available:

Þall copies, including the memory one, will be updated

Þthe updated block is sent to the requester processor (see figure b).

The resulting state of all copies will be shared.

Possible states of blocks in caches:

ÞValid-exclusive: The block is the only copy of the memory block. The cache and memory blocks are consistent.

ÞShared: There are several cached copies of the memory block and all of them are consistent.

ÞDirty: It is the only copy of the memory block and the memory block is inconsistent.

The state transition of the example protocol:

ÞP-Read and P-Write are commands (actions) initiated by the associated processor,

Þwhile Read-Blk is a consistency command arriving from the bus and initiated by other caches.

The state transition diagram defines:

how the cache controller should work when a request is given by the associated processor or by other caches through the bus.

For example:

a Read-Blk command arrives to a cache block in state Dirty Þ the cache controller should modify the state of the block to Shared.

State transition graph of the example protocol

Implementation:

The local caches are controlled by the local Cache controller, and the Snoopy controller.

The Snoopy controller monitors the bus-traffic and invalidates cache lines if necessary.

Example (previous slide):

The processor of PEi is executing an operation (store) updating a part of block A. The local cache-controller has detected a cache hit, updated the cache-line A, and started a bus cycle to update A in the memory.

The snoopy controller of PEj has recognized the write on the bus, and detected a hit for address A. Therefore it has to invalidate the local copy of block A.

It is similarly simple to implement a write-update policy. Using just more complex hardware, it is possible to grab not merely the address (for invalidation) but the data itself too from the bus.

Assessment:

+	simple

+	no essential change in processor architecture

	increased traffic of write-through policy

	bus is not scaleable

Directory Schemes

Coherence commands are not broadcasted, but sent selectively only to those nodes where valid copies of the shared data are stored.

Each node should know the places where further copies of its cached data are stored. This knowledge is stored in directory.

Directory:

	has an entry for each data block

	an entry is a list of node-identifiers where valid copies of the block are stored

Basic ideas to implement directories:

	Full-map directory

	Limited directory

	Chained directory

Full-map directory scheme

The centralized implementation of the directory:

	Presence Flag Vector (PFV) for each block in SM

	dirty bit: (C/D) stored with PFV

D (dirty): exactly one processor has write-permission (involves, 		that it stores the only-one valid copy)

C (clean): in any other case

	2 state bits in the processor's caches for each block: valid (V/I), write-permission (P/N)

Full-map directory scheme - 2

The protocol is based on the central role of the shared memory. Each change in states is initiated by nodes' requests, and coordinated by the memory.

Situations:

Read-hit and Write-hit with write-permission: no problem, no overhead

Replacement:	 invalid: replace

 valid, clean: replace and reset PFV flag

 valid, write-permission: write back, set clean, reset all 					PFV flags, replace

Read-miss:

	clean: load, set PVF flag

	dirty: the processor having write permission should write back, then 	load, set clean, and set PVF flag

Write-miss:

	clean: load, get write-permission, set dirty, write (see eg.)

	dirty: let the processor having write permission write back, then 	load, get write-permission, set dirty, write

Full-map directory scheme - 3

1.	Initial situation as in the previous Figure

2.	Cache controller of PEk detects Pk wants to write A, has no Wperm, sends WriteRequest to he Memory, blocks Pk, waits.

3.	Mem. has clean A, + flags in its PFV, sends Invalidate to owners, waits

4.	Receiving Invalidate, owners set valid flag of A to I, send ACK to Mem.

5,	Having all ACK-s, Mem sets D, resets PFV bits but PEk's, sends Wperm

6.	Having Wperm, PEk updates A and releases Pk

Limited directory scheme

The object of introducing:

èto reduce the bit overhead comparing to the full-map directory scheme.

Problem:

èonly a limited number of caches contained valid copies of the same data block

èmaintaining a presence flag vector is superfluous in most of the cases.

Solve:

èuse limited directories

Two main differences comparing to the full-map directory scheme:

èthe number of pointers (k) in a directory

entry << the number of processors (n) in the shared memory system

èthese pointers are dynamically allocated for those caches that contain a copy of the associated data block Þ simple bits are not enough to represent pointers.

In an n-processor system:

limited directory:

defining a presence pointer: Þlog2n bits

bit overhead: Þklog2n/block

full-map scheme: Þit is n/block.

klog2n < n

or

n

k < -----

log2n

If the number of valid copies of a data block <= k, Þthis protocol is very similar to the full-map scheme.

Else:

When cache Cn sends read request to the shared memory and all available pointer places in the limited directory entry are filled in with valid pointers, one of the copies of X in the caches should be invalidated and then the corresponding pointer could be replaced with a pointer to the requesting cache.

This is called eviction Þ selection policy /can be as simple as a pseudo random eviction policy/.

Chained directory scheme

The object of introducing:

ècompromise between the full-map and the limited directory scheme.

It preserves the limited nature of the central directory frames and even further restricts the number of presence pointers to one.

Each data block in each cache is extended with a pointer

Þto chain the valid copies of a data block.

Þas many presence pointers as many valid copies of the data block are created

Þno superfluous presence pointers in the directory entries of the shared memory

Figure (a): The first processor (currently P0) wants to read data block X Þthe memory sends a copy of X to C0, along with a special chain termination (CT) pointer Þ sets a pointer to C0 in the directory entry of X in the shared memory.

Figure (b): After receiving a second read request to X from cache C1, the memory sends a copy of X to C1, along with the previously stored pointer to C0, which is now replaced by a pointer to C1 in the directory entry of X. Cache C1 simply stores the incoming copy of X and the pointer to C0

Scaleable Coherent Interface

A concrete example for chained directory is the Scaleable Coherent Interface (SCI) defined by the IEEE standard 1596-1992,

Þ an interface to the interconnection network.

Characteristics:

àto provide a memory-address based, cache-coherent communication scheme for creating scaleable parallel machines with large numbers of processors.

àa scaleable linked list form of distributed directories.

àAny number of nodes can share the same memory line

àthe size of the directory is proportional to the number of cache lines.

àall SCI communications are organised in packets

àthe base protocol is a write-back and invalidate type

àorganisation is based on the so-called sharing-list in which each coherently cached block is chained up

The memory tag in each block of the memory is extended:

àthe identifier of the head node in the sharing-list (forw_id)

àa 2-bit memory state (mstate).

The cache tags of blocks are extended:

àidentifier for the previous node (back_id)

àidentifier for the following node (forw_id)

àcache state (cstate)

àthe mem_id field to identify the memory block.

Structure of sharing lists in the SCI:

Creation

Þa simplified version of the insertion operation when the associated sharing-list is empty.

Insertion:

Þwhen a cache miss occurs (at Ck in Figure), the node sends a prepend request to the memory,

Þthe memory responds with the pointer of the old head node,

Þit refreshes its head node pointer by the address of the requester,

Þafter receiving the response from the memory, the new head (if the sharing-list was not empty) notifies the old head node by sending a new-head request,

Þthe old head updates its backward pointer and returns its data.

Insertion in a sharing list

Main operations of the sharing list:

1. Creation

2. Insertion

3. Deletion

4. Reduction to a single-node

A node wants to remove its cache line from the sharing-list:

ÞFirst message:

It sends an update backward request containing its backward pointer (Pb) to its successor.

The successor will update its backward pointer by the value received in the request.

ÞSecond message:

It sends to the predecessor.

This message contains the forward pointer of the requester and is used by the predecessor to update its forward pointer.

Deletion from a sharing list

A cache line is written Þ all the other elements of the sharing-list must be invalidated.

Only the head node has the right:

àto update a cache line

àto remove the other elements from the sharing list

â

This is the reduction process:

ÞThe head sends a purge request to the second node of the list. Þ This node returns its forward pointer, i.e., the address of the next node in the list.

ÞThe head node sends a purge request to the third node,

Þand so on, until the last node receives the purge request.

ÞIf a node different from the head wants to execute a write on a shared cache line:

Þit has first to remove its cache line from the sharing-list

Þthen to insert it.

As a result the node becomes the head of the sharing-list having the right to update its cache line.

Reduction to a single-node

Hierarchical cache coherent protocols

These are applied in multiple bus based multiprocessors.

the single bus system Þ the hierarchical bus system

â

single bus based clusters or 'supernodes' are connected to a higher level bus via a higher level cache or 'supercache'

How multicache coherency control is achieved in hierarchical bus multiprocessors (shown Figure):

Assume:

Þprocessor P2 issues a write command to a data block X which has four copies in caches C11, C12, C16 and C17

Þcopies should be available in the higher level caches C20 and C22

A two level structure hierarchical cache coherence

How multicache coherency control is achieved in hierarchical bus multiprocessors (shown previous slide):

ÞGoodman's 'write-once' protocol is based on the write-invalidate policy Þ after writing X in C12 and C20, all the other copies of X should be invalidated.

ÞIt is achieved by broadcasting the write command:

Þon low level bus B10

Þthen on the higher level bus B20.

ÞThe copy of X in C11 is invalidated by the write command on bus B10.

ÞWhen the write command appears on bus B20 second level caches check if they have a copy of X.

â

When C22 detects a write command to X and finds a copy of X, it must invalidate its own copy, and

send down an invalidate command to bus B12 where copies of X in C16 and C17 should be invalidated.

ÞThe final result is that only the first and second level caches associated with the updating processor (C12 and C20) have copies of X.

ÞA subsequent read request issued by another processor (for example P5) should be broadcasted at the lower level and since, there is no copy at that level it should be propagated up the hierarchy.

ÞWhen C20 detects the read request for X and finds that it has a dirty copy of X Þ it issues a flush request down to bus B10 where cache C12 will relinquish exclusive access and supplies the dirty copy of X to cache C15.

Software-based protocols

Hardware-based protocols:

àAdvantage: the fastest mechanism of maintaining cache consistence

àDrawback: extra hardware complexity

Software-based protocols: a good and competitive compromise

All the software-based protocols rely on compiler assistance.

The compiler analysis the program and classifies the variables according to their use into one of the four classes:

1. Read-only

2. Read-only for any number of processes and read-write for one process

3. Read-write for one process

4. Read-write for any number of processes

Read-only variables can be cached without restrictions.

Type 2 variables can be cached only for that processor where the read-write process runs.

Since only one process uses type 3 variables it is sufficient to cache them only for that process.

Type 4 variables must not be cached in software-based schemes

At the end of each program section, the caches must be invalidated to ensure a consistent state of variables before starting a new section.

Methods of the invalidation:

1. Indiscriminate invalidation

2. Selective invalidation

The categorisation of software-based cache coherent protocols is shown in next slide.

Indiscriminate invalidation: Þ the complete cache is invalidated at the end of each program section. This scheme requires a single hardware mechanism, but it is very conservative.

Selective invalidation: Þ further categories according to the generation of program sections:

I. Critical section based scheme

II. Parallel for-loop based scheme

I. Critical section based scheme: Þ relies on the assumption that shared read-write variables are always accessed in critical sections guarded by software synchronisation tools. These variables are selectively invalidated when the critical section is completed.

II. Parallel for-loop based scheme:

A. Fast selective invalidation

B. Timestamp scheme

C. Version control scheme

A. Fast selective invalidation: Þ relies on a write-through cache policy and introduces three instructions to control cache accesses: Memory-Read, Cache-Read, and Cache-Invalidate.

B. Timestamp scheme: Þ allocates a clock to each data structure and a timestamp entry to each cache line. The clock associated with a data structure is updated at the end of each program section when the data structure was updated.

C. Version control scheme: Þ avoid unnecessary invalidations. Writing a variable is permitted only to one process in a program section and this writing generates a new version of the variable. When the writing process exits its program section, the new version becomes the current version that can be used by other processors, too.

Indiscriminate

invalidation

Synchronisation and event ordering in multiprocessors

Levels of exploiting parallelism in a multiprocessor:

ÞHigher level: the program is partitioned into processes that are executed on different processors Þ concurrent processing.

ÞLower level: memory accesses within a process could be executed in an overlapped way with other accesses in order to reduce load/store latency stemming from cache miss Þ hiding latency or access pipelining.

Another form of hiding latency Þ access buffering, when the memory accesses within a process are overlapped with other computations.

Special synchronising software tools:

ÞThe lowest level of these structures: the lock mechanism.

Synchronisation mechanisms in multiprocessors

ÞHigh level synchronisation language constructs /semaphores, conditional critical regions, monitors, etc./

It solves mutual exclusion and other synchronisation problems.

These shemes based on:

ÞLow level synchronisations tools realised or supported by hardware.

Criteria of cached test-and-test&set scheme:

1. minimum amount of traffic generated while waiting

2. low latency release of a waiting processor

3. low latency acquisition of a free lock

These schemes are moderately successful in small cache based systems like shared bus based multiprocessors.

Lock implementations in shared bus systems

Test&set synchronisation operation: Þ to support the implementation of high level language constructs.

Simple variable: 'lock' Þ CLOSED or OPEN.

test&set is an indivisible, atomic operation on the lock variable defined by the following C program:

char *lock;

while (exchange(lock, CLOSED) == CLOSED);

Lock implementations in shared bus systems / cont./

ÞDuring the execution of these two memory cycles the shared bus must not be granted to any other processor in the system.

ÞAfter finishing the exchange instruction, the LOCK control line enables again the bus arbiter that can now allocate the shared bus to another processor.

Problem: if the test&set operation was failed /the lock was in the state CLOSED/.

Solve: the processor should repeat the operation as long as the lock is CLOSED Þ a form of busy waiting:

Þties up the processor in an idle loop

Þincreases the shared bus traffic and

contention.

This type of lock Þ spin-lock Þ a significant cause of performance degradation when a large number of processes simultaneously use it.

Application of spin-locks Þ a new problem Þ thrashing

Assume that processor Pi successfully executes a test&set operation on lock. As a result, a dirty copy of lock remains in the cache Ci :

Example for trashing /continue/:

ÞSubsequent test&set operation on lock by Pj (see Figure b):

Þ cache Ci to send a copy of lock to Cj

Þboth copies Þ shared.

Þthe second phase of test&set should write lockÞwhich entails sending an invalidate command on the shared bus

Þand makes the copy of lock in Cj dirty

ÞPj finishes the test&set operation.

ÞPk wants to perform test&set on lock(see Figure c):

Þthe same bus operations are required

ÞCk contains a dirty copy of lock.

ÞSince the test&set operation of Pj was failed, Pj attempts again to execute test&set. Þ the block of lock is copied back to Cj Þ Cj and Ck play ping-pong with the block of lock as long as Pi remains in state CLOSED.

Lock implementations in shared bus systems / cont./

Problem was: Þ thrashing

Solve: for example:

A. Þ Snooping lock:

It relies on the same hardware support but defined by a different software algorithm:

while (exchange(lock, CLOSED) == CLOSED)

while (*lock == CLOSED);

Þexecuting the exchange operations by Pj and Pk

Þwhen Pj executes the *lock==CLOSED, the dirty block of lock from Ck is copied to Cj

Þboth blocks become shared

Þthe two copies can be read independently.

Advantage: Þ the introduction of the simplified second while loop which can be executed on the cached copy of lock without accessing the shared bus since it only reads the lock.

ÞThe ping-pong effect is solved.

Drawback: Þ when the locking processor releases the lock, the waiting processors compete for executing their exchange operations Þ a temporary bus saturation.

/This problem is not too serious if large critical sections are applied in the program./

B. Þ test-and-test&set scheme:

for (;;) {

while (*lock == CLOSED);

if (exchange(lock, CLOSED) != CLOSED)

break;

}

Advantage: Þ a newcomer processor does not disturb the waiting processors.

Drawback: Þ cannot prevent bus saturation when the lock is released.

â

Remedy:

Þ collision avoidance locks: enable the waiting processes to restart the exchange operation after a lock release with different delay time.

Þ tree of locks: the root lock protects the critical section. It is a tournament:

Þthe winner of a contest at a lower level becomes a contestant at the next level

Þfinally a process reaches the root lock

Þit wins the contest it has the permission to enter its critical session.

It is called tournament locks.

C. Þ queue lock: Þthe most effective lock type.

The structure of a queue lock is more complicated than a single byte representing CLOSED or OPEN state. Processes are waiting for the lock in a queue (represented by the lock array) from which they are served in a first-in-first-out way.

Lock implementations in scaleable multiprocessors

Problem: Þ collision /it is more serious in scaleable multiprocessors where hundreds of processors can compete for the same lock./

The memory location of lock becomes a hot spot Þ latency can enlarge in the interconnection network.

Solve: for example:

Þfetch&add: a new read-modify-write type primitive

/It was introduced in the NYU Ultracomputer and in the IBM RP3 multiprocessor./

fetch&add( x, a )

int *x, a;

{ int temp;

temp = *x;

*x = *x + a;

return( temp );

}

Þall the memory modules are augmented with an adder circuit Þ the variable x is locked only for two memory cycles /otherwise for 10-20 cycles/.

Þto avoid the hot spot problem the fetch&add operation can also be combined by the applied omega network Þ all switches are enhanced with adder circuits.

The original DOALL loop is as follows:

DOALL N=1 to 1000

<loop body using N>

ENDDO

The equivalent implementation by the fetch&add primitive is the following:

i = fetch&add(N,1);

while (i <= 1000) {

loop_body(i);

i = fetch&add(N,1);

}

Þafter initialising N to 1 each processor executes a fetch&add operation on N before entering the loop

Þthe fetch&add returns with different values that are used by the processors as index in the loop_body procedure

Þthe work of the combining network in realising the distributed fetch&add operation for 8 processors is depicted in the next slide.

Distributed execution of fetch&add in a combining omega network

Figure A: One of the incoming messages is stored in the switch while the other one is forwarded with an increment which is the sum of the original increments.

Event ordering in cache coherent systems

A. Strong ordering of events

Requirements of a correct solution of a parallel program execution:

Þsequential consistency: if any execution of a parallel program is equivalent with an interleaved execution on a uniprocessor system.

A necessary and sufficient condition for a system: Þ atomic memory accesses.

Memory accesses:

Þin parallel systems without caches are always atomic Þ strongly ordered for maintaining sequentially consistency

Þadding caches to the processors Þ non-atomic

Þin shared bus multiprocessors Þ memory accesses remain atomic

Problem: non-atomic memory accesses

Example: three processors want to access the memory cells A and B in the following order:

P1: ..., store A, ...

P2: ..., load A, store B, ...

P3: ..., load B, load A, ...

Assume that initially each processor has a read-only copy of A and B in its cache.

Step 1. P1: a store operation on A that invalidates the other copies.

Step 2.:

ÞP2: a read operation to get the new value of A since the invalidation message has already arrived to its cache.

ÞP1: writes back the new value of A into the memory

ÞP1: sends a copy to P2

ÞSince the load operation successfully finished, P2: a store operation on B that invalidates the copy of B in the cache of P3.

Step 3.

ÞP3 reads the new value of B as it was updated by P2 because the invalidation message of B has already arrived

ÞP3 reads the old value of A since the associated invalidation message has not yet reached P3.

If a hardware mechanism could support to restore the atomicity of memory accesses, sequential consistency could be satisfied in scalable multiprocessors, too.

Processor consistency: a slightly relaxed version of sequential consistency, but:

1. All previous loads must be performed before a load is allowed to perform with respect to any other processor.

2. All previous loads and stores must be performed before a store is allowed to perform with respect to any other processor.

Event ordering in cache coherent systems /cont./

B. Weak ordering of events

Weakly ordered systems: are able to further relax the conditions of event ordering by relating memory request ordering to synchronising points of the program.

Two consistency models:

Þthe weak consistency model:

Þthe release consistency model

Definitions:Þcompeting accesses

Þsynchronisation accesses

Þrequest accesses

Þrelease accesses

Þnon-synchronisation accesses

Þordinary accesses

Conditions of the week consistency:

1. All previous synchronisation accesses must be performed before an ordinary load or store access is allowed to perform with respect to any other processor.

2. All previous ordinary load and store accesses must be performed before a synchronisation access is allowed to perform with respect to any other processor.

3. Synchronisation accesses are sequentially consistent with respect to one another.

Conditions of the release consistency:

1. All previous request accesses must be performed before an ordinary load or store access is allowed to perform with respect to any other processor.

2. All previous ordinary load and store accesses must be performed before a release access is allowed to perform with respect to any other processor.

3. Competing accesses are processor consistent with respect to one another.

B. Weak ordering of events /cont/

The Figure below compares the week and release consistency models.

When a node of the graphs becomes executable, ordinary memory accesses (load/store) inside the nodes can be executed in any order without restrictions in both models.

The main difference between the week and release consistency models: the condition of enabling the execution of nodes in the graphs.

Event ordering in cache coherent systems /cont./

Implementation of memory consistency models:

(1) Inter-block access ordering, i.e. ordering accesses to distinct memory blocks

(2) Intra-block access ordering, i.e. ordering accesses to the same memory block

Multiprocessors with scaleable interconnection networks: Þ a special hardware mechanism Þ fence

/inter-block access ordering for release consistency/.

Fence operations:

Þfull fence delays /all future read and write accesses/

Þwrite fence delays /all future write accesses/

Þimmediate fence delays /only the access immediately following the fence/

Intra-block ordering of accesses Þ is necessary in an invalidation based cache coherent multiprocessor where invalidations may reach different processors at different times and acknowledge messages are necessary to indicate that the write operation has been finished.

There are two options to handle read and ownership requests to a block with pending invalidates:

(1) The request is delayed until the invalidations are complete. This solution results in an atomic store operation.

(2) The request is serviced but the requesting processor is notified that the acknowledge messages of the pending invalidates should be collected. Though this alternative does not guarantee atomicity of the store access, at least informs the requesting processor when the store has performed with respect to all processors.

The SCI cache coherence protocol supports both strong and weak ordering.

Uniform Memory Access (UMA) Machines

UMA architecture:Þnot suitable for scaleable parallel computers

Þexcellent for small size single bus based

multiprocessors

Two such machines:

ÞEncore Multimax

ÞEncore Computer Corporation

Þtechnology of the late eighties

ÞPower Challenge

ÞSilicon Graphics Computing Systems

Þtechnology of early nineties

Design space of single bus based multiprocessors:

(the main design differences between the two multiprocessors)

Encore Multimax /UMA/

Features:

ÞNanobus, which was one of the first commercial applications of a pended bus:

Þthe address and data buses are separated

Þthe address bus initiates both memory read and memory write transfers

ÞIn case of write transactions the data bus is used together with the address bus, while in a read transaction the data bus can be used by a memory unit to transfer the result of a previous read access.

Þseparate but co-operating arbiter logics:

Þit allocates the address and data bus among the 20 processors and 16 memory banks

Þround-robin arbitration policy for the address bus.

Þdata bus arbiter:

Þcentral arbiter

Þdistributed access control

The data bus arbiter applies a fixed priority arbitration policy among the memory controllers. The priority of a memory controller depends on its physical location on the Nanobus.

Þa 256 Kbyte cache with a bus watcher (snoopy controller) logic for each processor unit

Þthe bus watcher Þ write-through memory update policy and write-invalidate cache coherence policy

Þa special ownership scheme Þ to maintain cache coherency

Þthe application of pipelining both on the processor board and on the memory boards:

ÞPipelining enables the processor to start a new bus cycle before finishing the previous one, and for the memory controller to receive a new memory access request before completing the service of the previous one.

ÞPipelining is implemented by applying buffer registers on both the processor board and the memory board.

Power Challenge /UMA/

Features:

Þtwo versions:. The employed in both cases.

Þdeskside configuration, supporting up to six processors

Þrack configuration, supporting up to 19 processors and very large memory

Þprocessor: MIPS R8000 RISC processor:

Þthe parameters of its maximum system configuration:

ÞPOWERpath-2 split-transaction shared bus:

Þup to eight reads can be outstanding at any time

Þit was designed on the base of the RISC philosophy: each transaction requires exactly the same five bus cycles: arbitration, resolution, address, decode, acknowledge.

Þ40-bit address bus and the 256-bit data bus are arbitrated separately

Þbus transactions:

| Bus transaction | Address bus | Data bus |
| --- | --- | --- |
| write request | + | + |
| read request | + | - |
| invalidate request | + | - |
| read response | - | + |

| Component | Description |
| --- | --- |
| Processor | 19 MIPS R8000, 5.4 Gflops peak |
| Main memory | 16 Gbytes, 8-way interleaving |
| I/O bus | 4 POWERchannel-2, each 320 Mbytes/sec |
| SCSI channels | 32 fast-wide independent SCSI-2 channels |
| Priority groups of bus transactions: 1.read responses 2.address operations (read requests, invalidate requests, and interrupts 3.write requests Disk | 3.0 TByte disk (RAID) or 960 Gbyte disk |
| Connectivity | 4 HIPPI channels, 8 ethernet channels |
| VME slots | 5 VME64 expansion buses provide 25 VME64 slots |

Power Challenge /UMA/

Features:

ÞPOWERpath-2 bus /cont./

Þround-robin arbiter policy

Þdistributed arbitration scheme

Þless transactions on the bus

Þreduces latency

Þwrite-invalidate cache coherence by snoopy hardware

Þcache coherence policy: Þ a slightly modified version of the Illinois Protocol:

Each cache line has four states: Invalid, Exclusive, Dirty exclusive, Shared.

Þmemory update policy: Þ write-back:

Þa read request is satisfied from another processor's cache Þthe cache read response is used to update the memory as a writeback.

Similarly to the Encore Multimax, the processor-bus interface maintains a duplicate set of cache tags to reduce contention between the processor and the snoopy controller.

Þ

Þ

Þ

ÞBarrier synchronisation:

ÞFirst activity: N processes are started by a fork operation

Þat the end of the parallel activity the N participating process should wait for each other in a join operation

Þspecial hardware mechanism and compiler technique supports it: Þ SGI's parallel compilers

To avoid N read accesses to the start bit the POWERpath-2 bus provides a piggyback read. This means that any number of processors can participate in a single read transaction by accepting the read response as their own and indicating that the corresponding cache block is shared.

Þthe join operation is implemented by a counter initialised to N at forking.

If a processor finishes its activity, it decrements the counter. The join operation is completed when the counter reaches 0.

To reduce the bus traffic to only N bus transfers, the join mechanism is supported by a special count register at each processor interface.

Non-Uniform Memory Access (NUMA) Machines

Characteristics of NUMA Machines:

Þavoid the memory access bottleneck of UMA machines

Þlogically shared memory is physically distributed

Þhighly scaleable

Þvery sensitive of data allocation in local memories

Þaccessing a local memory segment of a node is much faster than accessing a remote memory segment.

ÞThe problem of cache coherency does not appear in distributed memory multicomputers since the message passing paradigm explicitly handles different copies of the same data structure in the form of independent messages.

Þthe hardware supported cache consistency schemes are not introduced into the NUMA machines

These systems cache read-only code and data, as well as local data, but not shared modifiable data. This is the distinguishing feature between NUMA and CC-NUMA multiprocessors.

ÞNUMA machines are closer to the multicomputers than to other shared memory multiprocessors

Typical NUMA machines:

ÞBBN TC2000

ÞIBM RP3

ÞCray T3D

ÞHector multiprocessor

Hector /NUMA/

Features:

Þhierarchical NUMA machine consisting of stations connected by a hierarchy of ring networks

ÞStations:

Þsymmetric multiprocessors

Þthe processing modules (nodes) are connected by a single bus

ÞNodes:

Þprocessor/cache unit

Þmemory unit

Þstation bus interface which connects the otherwise separated processor and memory buses /shown in next slide/

The separation of the two buses enables other processors to access this memory while the processor performs memory access operation in off-node memory.

Þa flat, global address space Þ each processing module is assigned a range of addresses

the addressing scheme Þr+s+p bits

/ r identifies the ring, s points to the station, and p addresses the slot inside the station/

Þa snoopy protocol provides cache consistency among the nodes inside a station.

ÞMemory accesses:

synchronised packet-transfer scheme controlled by a hierarchy of interface circuits:

1. Station bus interface:

It serves for connecting processing modules to the station bus by forwarding station bus requests to the station controller

2. Station controller (local ring interface):

a./ it controls the allocation of the station bus between on-station requests

b./ it realises the local ring interface for the station.

3. Inter-ring interface:

two-deep FIFO buffer that gives priority to packets moving in the global ring

Main advantages of the Hector machine:

1.The hierarchical structure enables short trans-mission lines and good scalability.

2.The cost and the overall bandwidth of the structure grow linearly with the number of nodes.

3.The cost of a memory access grows incrementally with the distance between the processor and memory location.

Main drawbacks:

1.the lack of global cache consistency

2.the nonuniform memory access time which require careful software design.

Structure of the Hector machine

Cray T3D /NUMA/

Features:

Þshared memory and the message passing programming paradigms

Þthe shared memory is distributed among the processing elements

Þno hardware support for cache coherency

Þspecial software package and programming model - the CRAFT - manages coherence and guarantees the integrity of the data.

Þhardware structure:

1.microarchitecture: /the node organisation/

ÞDigital's 21064 Alpha AXP microprocessor:

Main weaknesses:

1. Limited address space

2. Little or no latency-hiding capability

3. Few or no synchronisation primitives

Þup to 128 gigabytes distributed shared memory that requires at least 37 bits of physical address

Þa 32-entry register set

Þto improve the latency hiding mechanism of the Alpha chip, Cray introduces a 16 word length FIFO Þ prefetch queue Þ  permits to perform 16 prefetch instructions without executing any load from the queue.

Þsupports 4 synchronisation mechanisms by hardware.

Þthe barrier hardware comprises 16 parallel logical AND trees that enable multiple barriers to be pipelined

Þfetch-and-increment hardware

/ specialised register set/

2.macroarchitecture: /how to connect and integrate the nodes of the parallel computer./

Þmemory system:

·distributed shared memory where any PE can directly address any other PE's memory;

·each PE contains 16 or 64 Mbytes of local DRAM. Access time of the local memory is between 13 and 38 clock cycles (87 to 253 nsec).

·latency for accessing remote memory varies 1~2 microseconds

·write-through, direct-mapped, read-allocate cache technique

Þinterconnection network

Þtopology: 3D torus

Þrouter logic: a dimension-order, wormhole routing algorithm with four virtual channels

Þthe network moves data in one or four 64-bit word length of packets.

Cache-Coherent Non-Uniform Memory Access

(CC-NUMA) Machines

Characteristics of CC-NUMA Machines:

Þscaleable shared memory multiprocessor

Typical CC-NUMA machines:

ÞVisconsin Multicube

/the snoopy cache protocol but in a hierarchical way/

ÞAquarius Multi-Multi

/combines the snoopy cache protocol with a directory scheme, but the interconnection network Þ the shared multibus concept/

ÞStanford Dash/clusters,

combines the snoopy cache protocol with a directory scheme /

ÞStanford FLASH/integration of cache-coherent shared memory with high-performance message passing,

directory scheme for maintaining cache coherence/

ÞConvex Exemplar

/Scaleable Coherent Interface,

directory scheme based global cache coherence/

Main features of CC-NUMA machines:

| Machine | Complexity of nodes | Memory distribution | Cache consistency scheme | Interconnection network |
| --- | --- | --- | --- | --- |
| Wisconsin Multicube | single processor | per column bus | snoopy cache | grid of buses |
| Aquarius Multi-Multi | single processor | per node | snoopy cache + directory scheme | grid of buses |
| Stanford Dash | bus based multiprocessor (cluster) | per cluster | snoopy cache + directory scheme | -bus inside cluster -pair of meshes among clusters |
| Stanford FLASH | single processor | per node | directory scheme | 2D mesh |
| Convex Exemplar | hypernode (8 processors) | per hypernode | directory scheme (Scaleable Coherent Interface) | - crossbar inside hypernode - multiple unidirectional rings among hypernodes |