Wisconsin multicube /CC-NUMA/

Features:

Þarchitecture:

Þrow and column buses forming a two-dimensional grid architecture:

Þthe three-dimensional generalisation will result in a cube architecture.

Þthe main memory is distributed along the column buses, each data block of memory has a home column

Þeach row of processors works similarly to single bus based multiprocessors

Þeach processing element:

Þa processor

Þa conventional cache memory to reduce memory latency

Þa snoopy cache that monitors a row bus and a column bus in order to realise a write-back, write-invalidate cache coherence protocol

ÞIn order to describe the cache coherence protocol, the following definitions should be given:

1.Definition of possible states of blocks in caches, and memories.

2.Definition of commands to be performed at various read/write hit/miss actions.

3.Definition of state transitions in caches, and memories according to the commands.

4.Definition of transmission routes of commands among processors, caches, and memories.

Wisconsin multicube /CC-NUMA/

Features /cont./:

1.a Possible states of blocks in memories:

A. unmodified:

value in the main memory Þ correct

it can have several correct cached copies

B. modified:

value in the main memory Þ stale

exists exactly one correct cached copy.

1.b Possible states of blocks in caches:

Þshared (its copy in the main memory is in global unmodified state),

Þmodified (its copy in the main memory is in global modified state),

Þinvalid

2. A cache controller can issue four types of consistency commands:

A.READ: the associated processor wishes to read a data block that is not present in its cache.

B.READ-MOD: the associated processor wishes to write a data block that is not in modified state in its cache.

C.ALLOCATE: an entire block is to be written regardless of its current contents.

D.WRITE-BACK: the data block in the main memory should be updated and set into unmodified state.

2.A.a READ request for a data block X which is in state modified:

-the READ request is issued on the row bus

-accepted by the controller whose modified line table contains the address of X.

-assume the requester is C00,

the modified copy of X is held by C11,

the home column of X is the column 2

/shown in next slide/:

nC01 accepts the request and broadcasts it on the column bus à all controllers change the state of X into unmodified.

nIn parallel, C11 picks up the request and fetches X.

nFinally, C11 transmits X back on the column bus where C01 accepts the data block and broadcasts it on the row bus.

nThe requesting cache C00 picks up X along with the home column controller C02 which writes X back to the memory in unmodified state.

Wisconsin multicube /CC-NUMA/ /cont./

The mechanism of reading block X in state modified:

Wisconsin multicube /CC-NUMA/

Features /cont./:

2.A.b READ request for a data block X which is in state unmodified:

nthe home column controller C02 accepts the request

nif X has a copy in C02 à it is directly sent back to C00 on the row bus

notherwise, C02 transmits the request to the memory on the column bus

nreceiving X from the memory on the column bus, C02 transmits X to C00 via the row bus.

2.B.a READ-MOD request for a data block X which is in state modified:

nAfter the first two bus operations (same as in READ request) C11 picks up the request, invalidates the copy of X and transmits it on the row bus

nHere, controller C10 accepts the data block and broadcasts it on the columns bus.

nFinally, C00 picks up X, updates and stores it in modified state

The main memory is not updated (write-back policy).

2.B.b READ-MOD request for a data block X which is in state unmodified:

nThe home column controller C02 accepts the request and forwards it to the main memory which sends back an invalidate request along with a copy of X.

nEach controller on the column bus accepts the invalidate request and broadcasts it on the associated row bus.

nC02 broadcasts the invalidate request along with the copy of the block containing X on its row bus.

nFinally, C00 picks up the copy of X and broadcasts a consistency command on its column bus to force the other caches to insert the address of X to their modified line tables.

Stanford DASH /CC-NUMA/

Features:

Þnodes of Dash architecture:

Þsingle bus based multiprocessors à clusters

Þeach cluster:

Þfour processor pairs (MIPS R3000 processor and R3010 floating-point coprocessor)

ÞI/O interface

Þfirst- and second-level caches

/second level caches: write-back,

snoopy cache protocol/

Þa partition of the main memory

Þa directory

/cache coherent directory scheme/

Þintercluster interface logicÞ

Þinterconnection network: any low-latency direct network developed for message-passing architectures

In the concrete Dash prototype a pair of wormhole routed meshes is employed /shown in the next slide/. One mesh Þ for transmitting request,

other mesh Þ reply messages

Þdistributed implementation of the directory based cache coherence scheme:

The memory system of the Dash is partitioned into four levels /shown in the next two slides: a, b, c, d/:

Þprocessor level: can be exploited if the request can be serviced from the requester processor's cache /see Figure a/

Þlocal cluster level: when the request can be serviced from a cache inside the cluster /see Figure b/

Þhome cluster level: if the request cannot be serviced from the local cluster it is forwarded to the home cluster where the physical memory and directory of the given address is stored /see Figure c/

Þremote cluster level: if even the home cluster is unable to service the request.

This level consists of the caches of clusters marked by the home directory as holding a copy of the requested block /see Figure d/

Structure of the Dash machine:

Two 2 meshes

request, response)

Wormhole routing

120 MB/s/link

50 ns/nop

Nodes (clusters)

N0

N15

. . .

Cache (snoopy)

Processor

Network interface

Directory

Memory global addressing

4x MIPS R300

(33 MHz)

Snoopy bus

Modified Silicon graphics Power Station 4D/340

Node

Cluster

Node

Cluster

Node

Cluster

Node

Cluster

Request Mesh

Reply Mesh

(a) The prototype node implementation

(b) Block diagram of a 2x2 mesh interconnect

The Stanford Dash prototype system

Processor Level

Processor cache

Local Cluster Level

Other processor cache

within local cluster

Directory Home Level

Directory and main memory associated with given address

Remote Cluster Level

Processor caches in

remote clusters

(c) Logic memory hierarchy

ACCESS TIME

1 clock

30 clocks

100 clocks

135 clocks

The Stanford Dash prototype system

invalidation based

cache coherence protocol

Uncached

Shared

Dirty

Directory info on each mem. block:

- state

- clusters holding a copy

Stanford DASH /CC-NUMA/

Features /cont./:

ÞDash directory logic:

Þoutbound part à directory controller (DC)

Þforwarding local requests to remote clusters

Þreplying to remote requests

Þresponding to cluster bus with directory information

Þstoring locks and lock queues

Þinbound part à reply controller (RC):

Þremote access cache (RAC)

Þstoring state of ongoing memory requests and remote replies

Þsnooping on the bus

/128-Kbyte direct-mapped snoopy cache with 16-byte cache lines/

Þforwarding remote requests to the cluster bus

Þissuing cache line invalidations and lock grants

Þpseudo-CPU (PCPU)

Þhandler of the Y-dimension of the inter-connection network

The main functions of the directory logic during a read request

/shown the figure d., the circled numbers/:

1.Local cluster: An RAC entry is allocated and DC sends a Read-Req message to the home cluster.

2.Home cluster: PCPU issues a read on the cluster bus. Since the directory entry is in dirty state, DC forwards the Read-Req message to the dirty remote cluster.

3.Remote cluster: PCPU issues a read on the cluster bus.

4.Remote cluster: The cache data is placed on the cluster bus by the dirty cache. DC sends a Read-Rply message to the local cluster and a Sharing-Writeback to the home cluster.

5.Local cluster: RC gets reply and releases CPU's arbitration. CPU retries read and RAC responds with data.

6.Home cluster: PCPU issues Sharing-Writeback on the cluster bus. DC updates directory entry to shared state.

1.Local cluster: CPU's write buffer    àread-exclusive on cluster bus and is forced to retry. An RAC entry is allocated and DC sends Read-Ex Req to the home cluster.

2.Home cluster: PCPU àread-exclusive on cluster bus. The directory entry is in shared state, àDC sends Inv-Req message to all shared clusters and a Read-Ex Reply with data and invalidate counter to local cluster. DC updates directory state to dirty.

3.Remote cluster(s): PCPU àread-exclusive on cluster bus to invalidate shared copies. DC sends Inv-Ack to requesting cluster.

4.Local cluster: RC receives Read-Ex Reply with data and invalidation counter. It releases CPU's arbitration. Write buffer repeats read-exclusive and RAC responds with data. Write buffer retries write.

5.Local cluster: The invalidate counter of the RAC entry is decremented by each Inv-Ack. When the counter reaches 0, RAC entry is deallocated.

Sequence of actions in a store operation requiring remote service

Stanford FLASH /CC-NUMA/

Features:

Þefficient combination of directory based cache coherent shared memory architectures and state-of-the-art message-passing architectures

àreduce:

Þ the high hardware overhead of distributed shared memory machines

Þ the high software overhead of the multicomputers

General structure of FLASH and its node:

Stan

Þhigh-performance commodity microprocessor (MIPS T5):

Þcaches

Þportion of the main memory

ÞMAGIC chip:

Þmemory controller

Þnetwork interface

Þprogrammable protocol processor

ÞI/O controller

Þdirectory-based cache coherence protocol:

Þdirectory data structure

Þhandlers to realise the cache coherent protocol

Stanford FLASH /CC-NUMA/

Features /cont./:

Þmessage passing protocols support different message types

/long messages, short messages/

Structure of the MAGIC node controller:

Stan

ÞThe MAGIC node controller:

Þseparates data and control paths

Þthe Message Split Unit divides the incoming messages to header and data part:

Þheader is directed to the Control Macropipeline while the data part is handled by the Data Transfer Logic /16 databuffer/

Þcopying the data part is avoided by keeping it in the same buffer as it was received until the header is processed by the control pipeline.

Þthe output header and the data part are combined again to form an outgoing message

Stanford FLASH /CC-NUMA/

Features /cont./:

ÞControl Macropipeline /three stages/:

ÞInbox: à executes hardware dispatch for message headers and initiates speculative memory operations to reduce data access time

ÞProtocol Processor: here the actual message handler is executed;

its instructions and data are stored in instruction and data caches.

ÞOutbox: directs message headers to the destination places either to the network or to the processor or to the I/O subsystem.

Structure of the control macropipeline:

Stan

Convex Exemplar /CC-NUMA/

Features:

Type: SPP1000 /SPP = Scaleable Parallel Processor/

Þup to 128 processors

Þnodes:à symmetric multiprocessors:à hypernode:

Þfour functional blocks:

Þ2 CPUs (HP PA-RISC each) sharing a single CPU agent, and a memory unit holding hypernode-private memory data, global memory data, and network cache data

Þan I/O subsystem

Convex Exemplar Architecture:

Stan

Convex Exemplar /CC-NUMA/

Features /cont./:

Þhypernodes are connected by four SCI point-to-point, unidirectional rings

Þglobal shared memory is distributed among the hypernodes.

Þhierarchical memory architecture:

ÞCPU-private memory serves for data accessed only by a single CPU.

ÞHypernode-private memory is provided for data shared only by CPUs within a single Hypernode.

ÞNear-shared memory is globally accessible from all hypernodes. This memory is allocated from the global memory. Accessing a near-shared memory from the home hypernode requires less latency than accessing it from other nodes.

ÞFar-shared memory is globally accessible from all hypernodes with equal latency. It is allocated from the global memories of several hypernodes. Coherent memory that is designated far-shared (shared by and distributed across multiple hypernodes) is interleaved across the hypernodes on a per-page basis (4 Kbytes) by operating system software allocation of the table entries.

Þtwo domains: are bridged by a special network cache that tracks coherency in both domains

Þintrahypernode domain implements coherency within a hypernode's processors and their memories

Þinterhypernode domain implements coherency among the hypernodes of the system

Þwrite-back, invalidate SCI coherency protocol

Convex Exemplar /CC-NUMA/

Features /cont./:

The cache coherence protocol of the intrahypernode domain satisfies the following requirements:

1.Any number of read encachements of a cache line may be made at the same time. The cache line may be read-shared in multiple caches.

2.To write data into a cache line, the cache line must be owned exclusively. This implies that any other copies must be deleted.

3.Modified cache lines must be written back to memory before being deleted, or the contents can be forwarded directly to the new owner.

Pseudo-code of a simple matrix multiply /example for above rules/ :

The algorithm calls spawn to fork nCPUs number of threads.

àEach thread receives a unique thread identifier, itid.

àEach iteration of the outer do j... loop is only executed by one CPU as determined by the following if statement.

àThe CPUs are allocated in cyclical fashion to the loop iterations.

Parallel matrix multiply code for cache-coherent machine:

global c(idim,idim), a(idim,idim)

global b(idim,idim), nCPUs

private i, j, k, itid

call spawn( nCPUs )

do j = 1, idim

if ( jmod(j,nCPU).eq.itid ) then

do i = 1, idim

c(i,j) = 0.0

do k = 1, idim

c(i,j) = c(i,j) + a(i,k) * b(k,j)

enddo

enddo

endif

enddo

call join

Convex Exemplar /CC-NUMA/

Parallel matrix multiply code for noncache-coherent machine /example from the previous slide/:

global c(idim,idim), a(idim,idim)

global b(idim,idim), nCPUs

private i, j, k, itid, tmp

semaphore is(idim,idim)

call spawn( nCPUs )

do j = 1, idim

if ( jmod(j,nCPU).eq.itid ) then

do i = 1, idim

tmp = 0.0

do k = 1, idim

call flush(a(i,k))

call flush(b(k,j))

tmp = tmp + a(i,k) * b(k,j)

enddo

call lock(c(i,j), is(i,j))

c(i,j) = tmp

call flush(c(i,j))

call unlock(c(i,j), is(i,j))

enddo

endif

enddo

call join

àThe calls to flush on the a and b arrays are required to ensure that the executing CPU gets an up-to-date copy of the input arrays, because they may have been modified on a different CPU after being encached on the current CPU.

àThe call to flush on the c array is needed to write the output array back to memory, so that subsequent operations will see the result of the matrix multiply.

Cache--Only Memory Architectures (COMA) Machines

Characteristics of COMA Machines:

Þtry to avoid the problems of static memory allocation:

Þ excluding main memory blocks from the local memory of nodes

Þ employing only large caches as node memories

Þrender static data allocation to local memories superfluous

Þdata allocation is demand driven, according to the cache coherence scheme, data are always attracted to the local (cache) memory where they are needed

Þscaleable parallel architectures

Þdirectory schemes and hierarchical cache coherent schemes

Typical COMA machines:

ÞDDM (Data Diffusion Machine)

/hierarchical cache coherence scheme/

ÞKSR1

/distributed directory/

DDM (Data Diffusion Machine)

Þhierarchical, tree-like multiprocessor, where the leaves of the tree represent the basic DDM architecture

Þsingle-bus based multiprocessor

Þattraction memory:

1. State and data memory unit

2. Controller

3. Output buffer

Þtwo protocols /in the controller/:

Þthe snooping memory above protocol: à the cache coherence scheme among the attraction memories

Þthe memory below protocol à the interface between the processor and the attraction memory

Both protocols consider a cache line of the attraction memory as one unit and a state field is stored in the attraction memory for each unit.

Structure of the basic DDM machine

Þ Write-invalidate snoopy-cache protocol

Cache line can occur in any of the following seven states:

1.I (Invalid): This attraction memory does not contain the cache line.

2.E (Exclusive): This attraction memory contains the cache line and there is no other copy in the system.

3.S (Shared): There are several copies of the cache line and one of them is located in this attraction memory.

4.R (Reading): This node started a read transaction and is waiting for the reply.

5.W (Waiting): This node started an erase transaction and waits for becoming Exclusive.

6.RW (Reading-and-Waiting): This node is waiting for a data value that becomes Exclusive later.

7.A (Answering): This node is supposed to answer a read request.

DDM /cont./

The following transactions are permitted on the DDM bus:

1.Erase: Erases all copies of this cache line.

2.Exclusive: Acknowledges an erase request.

3.Read: Reads a copy of the cache line.

4.Data: Transfers data for an earlier read request.

5.Inject: Carries the only copy of a cache line and looks for an attraction memory to place it. (Inject is initiated by a replacement.)

6.Out: Transfers the cache line out of the attraction memory. It will terminate when another copy of the cache line is found. (Out is caused by a replacement.)

State transition diagram of the attraction memory protocol:

DDM /cont./

The single-bus based basic DDM architecture à

hierarchical multiprocessor system:

replacing the top protocol with a directory unit

The directory unit can be used to connect two higher level buses.

Write race in the hierarchical DDM:

DDM /cont./

Directory unit:

ÞThe most important part:

set-associative state memory:

Þ administrative information for each cache line

Þ the state memory does not contain any data

Þcontroller realises two protocols:

Þ directory above protocol: a snooping one /resembling very much to the memory above protocol/

Þ directory below protocol: compatible with the top protocol for items in Exclusive state.

Structure of the directory unit:

KSR1

ÞCOMA Machine

Þlogically single address space:

Þ local caches

Þ ALLCACHE Engine: a sequentially consistent cache coherence algorithm based on a distributed directory scheme

It stores data in pages and subpages.

The unit of memory allocation in local caches is the page containing 16 Kbytes.

The unit of transferring data among local caches is the subpage consisting of 128 bytes.

Þdistributed directory:

a matrix:

Þ a row is allocated for each subpage

Þ a column for each local cache

A matrix element is empty if the corresponding local cache does not contain any copy of the subpage.

Nonempty elements can represent any of the following four states:

EO (Exclusive Owner): This is the only valid copy in the whole machine.

C (Copy): At least two valid copies of the subpage exist in the machine.

NO (Non-exclusive Owner): When several copies of the subpage exist, one of them is marked as the non-exclusive owner.

I (Invalid): Though a copy of the subpage exists in the local cache, it is invalid and will not be used.

Read miss:

Þsending a request packet to the ALLCACHE Engine

Þthat sends back a copy of the requested data from the local cache where the exclusive owner is currently situated

Þat the same time the state of the owner is changed to Non-exclusive Owner

Þif there is no exclusive owner, the copy of the requested data is taken from any local cache where a valid copy exists

/shown the next slide/

ALLCACHE Engine operations in the KSR1

KSR1 /cont./

Þ hierarchical

Þ organisation based on rings (see below):

Þlowest level: Þ ALLCACHE Group:0s

Each group at this level consists of an ALLCACHE Engine:0 and a set of local caches

Þnext level: ALLCACHE Group:0s are combined with an ALLCACHE Engine:1 composing an ALLCACHE Group:1 and, so on

ALLCACHE Engine:i contains directory information about all the cache lines belonging to ALLCACHE Group:i.

The hierarchical structure of the KSR1 machine

Comparing the dynamic and static network characteristics

| Network | Bus | Crossbar | Multistage | Tree |  | Hypercube |  | Mesh |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Minimum latency | const. | const. | log(N) |  | log(N) |  | log(N) | N1/2 |
| Bandwidth per processor | 1/N | const. | const. |  | const. |  | const. | const. |
| Wiring cost | w (width of bus) | N | Nlog(N) |  | N |  | Nlog(N) | N |
| Switching cost | N | N2 | Nlog(N) |  | --- |  | --- | --- |
| Connectivity | any to any but only one to one at a time | all permutations, one at a time | some permutations, if the network is unblocked |  | 3 nearest neighbours |  | log(N) nearest neighbours | 4 nearest neighbours |

|  | VSM - Virtual Shared Memory | EMP - Explicit Message Passing | RISC-standard CPU | 64-bit addressing | cache coherence | compatible to WS surface | WS binary interface | all 64-bit data | CRAY-/C-series compatible | standalone single OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CONVEX SPP | Y | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| CRAY T3D | Y | Y | Y | Y | no | no | no | Y | Y | no |
| IBM SP-1/2 | no | Y | Y | no | no | Y | Y | Y | no | no |
| INTEL Paragon | no | Y | no | no | no | no | no | no | no | Y |
| KSR-1/2 | Y | Y | no | Y | Y | no | no | Y | no | Y |
| TMC CM5 | no | Y | (Y)* | no | no | no | no | no | no | Y |
| *Sparc+vector processor |  |  |  |  |  |  |  |  |  |  |